----------------------------------- Page 1 -----------------------------------

BBLIST.EXE (v3.15, 11/01) User's Manual

Copyright (C) 1993-2001 by RD Instruments - All rights reserved

---------------------------------------------------------------

RD Instruments   ----   Acoustic Doppler Current Profilers

9855 Businesspark Avenue,   San Diego,   CA,  USA,   92131

Tel: (858) 693-1178    Fax: (858) 695-1459    Tlx: 46-7731

Internet: http:/www.rdinstruments.com

------------------------------------------------------------------------------

"COPY BBLIST.DOC PRN" prints this 16-page manual.

------------------------------------------------------------------------------

Command syntax:

BBLIST [drive:][path][filename.ext] [/C] [/M] [/+] [/50] [?]

drive:       = drive containing BBADCP binary data files

path         = DOS directory path to BBADCP binary data files

filename.ext = file name and extension of BBADCP binary data file to load

/C           = use color screens

/M           = use monochrome screens

/+           = force decoding of ensembles with bad checksums

/50          = use 50-line/43-line video mode

?            = list command syntax

------------------------------------------------------------------------------

1 - INTRODUCTION

----------------

This printable/on-line manual explains how to run our BBLIST.EXE binary-to-

ASCII data display and conversion program.  BBLIST takes the binary data files

created by our Broadband Acoustic Doppler Current Profiler (BBADCP) and lets

you convert selected data fields to common units of measurement in an ASCII-

text format.  You can then use the ASCII files in programs that accept this

format.  BBLIST NEVER alters your original BBADCP binary data files.  BBLIST

lets you:

- Set processing parameters.

- Transform velocity data.

- Select the display/conversion limitations (e.g., data fields, bin range,

ensemble range, output file size).

- Select the BBADCP data fields to display or convert.

- Monitor and control the conversion process.

- View this manual when you need on-line help.

NOTE:  THIS DOCUMENT IS A BRIEF OVERVIEW OF THE BBLIST PROGRAM.  IT IS NOT A

TUTORIAL FOR BBADCP COMMANDS OR OUTPUT DATA FIELDS.  FOR A DETAILED EXPLANA-

TION, SEE APPENDIX C (COMMANDS) AND APPENDIX D (BBADCP OUTPUT DATA FORMATS) IN

THE BBADCP TECHNICAL MANUAL.

2 - INSTALLING AND STARTING BBLIST

----------------------------------

BBLIST requires an IBM-compatible computer, preferably with a hard drive for

faster data conversions.  If you are familiar with software programs, you can

easily start using BBLIST.  Copy BBLIST.EXE and BBLIST.DOC from the utility

disk to the directory that has the BBADCP binary data files (or put BBLIST in

a DOS directory path).  You do not need BBLIST.DOC if memory/disk space is

limited.  The BBLIST command format is at the top of this manual.  Examples of

this format are:

----------------------------------- Page 2 -----------------------------------

- "BBLIST" <Enter> - Starts BBLIST and lets you select the BBADCP binary

data file from within the program.

- "BBLIST C:\BBDATA\TEST.000 /M" <Enter> - Starts BBLIST and loads the

TEST.000 file located on the C: drive in the BBDATA directory.  The /M

forces BBLIST to use monochrome screens.

When you run the program the first time, BBLIST displays its introduction and

copyright screen.  During operation, BBLIST creates/updates a pointer file

named BBLIST.PTR.  This file saves the current working directory, the name of

the last binary file used, the name of the format file (*.FMT) last used, and

the color selection.  This may help save you time by automatically calling up

the files you used last.  If you need to see the introduction screen again,

you must first delete or rename the BBLIST.PTR file.

3 - USING BBLIST

----------------

Sections 4 through 7 use the main menu (first line on each screen) to guide

you through the steps needed to convert a BBADCP binary data set to an ASCII

data set.  We assume you have already started BBLIST.  First, though, you must

learn how to move around in BBLIST.  Most keystrokes are like those in other

popular programs.

- Select a menu item with the highlighted letter or cursor control keys

(up/down, left/right, PgUp/PgDn, Home/End).  <Alt-first letter> lets you

instantly access a main menu item.

- <Enter> selects an item or accepts an entry.

- <Spacebar> cycles through menu items that have more than one option.

- <Esc> aborts, cancels, exits, or "backs out."

- <F1> shows the on-line help manual with its text search (FIND) feature.

- <F2> saves all the current menu settings to a format file (par. 4-1).

- <F3> loads a binary data file or format file (par. 4-2).

- <F5> removes spaces from the left side of a conversion field.

- <F6> adds spaces to the left side of a conversion field.

- <F7> removes spaces/decimals from the right side of a conversion field.

- <F8> adds spaces/decimals to the right side of a conversion field.

- <F9> toggles between a view of the converted data and the layout screen.

- <F10> switches between the menu bar and the display screen.

- <+>/<->/<J>ump keys move to next/previous ensembles on display screens.

- <Tab>/<Shift-Tab> scrolls left/right on the converted data screen.

- <Alt-X> exits the program.

- Entry editing keys are Ins, Del, Backspace, and the cursor control keys;

<Ctrl-Enter> accepts an entry up to, but not including, the cursor.

4 - FILE MENU

-------------

Use this menu to save your menu settings and conversion layout selections to a

configuration format file (*.FMT), to load a BBADCP binary data file or format

file, to display the file information screen, or to exit BBLIST.

4-1.  SAVE <F2>.  Selecting this menu item, or pressing <F2> in any menu, lets

you save all the current menu settings to a format file (*.FMT).  Use standard

DOS syntax for file names/paths.  Do not include an extension; BBLIST always

adds the FMT extension.  The next time you run BBLIST, the BBLIST.PTR file

automatically loads the format file used last if you start the program from

the same directory.

----------------------------------- Page 3 -----------------------------------

4-2.  LOAD <F3>.  Selecting this menu item, or pressing <F3> in any menu, lets

you load a BBADCP binary data file or a previously-saved format file (*.FMT).

This menu uses DOS syntax for names/paths.  "*.*" lists all files in the pres-

ent directory.  BBLIST shows parent directories as "..\" and subdirectory

names with a trailing "\".  As a shortcut, you can try using "*.0*" to locate

binary data files because most binary data files follow this pattern.  Try

using "*.FMT" to locate a format file.

4-2.1.  Binary Data.  If you did not include a binary data file name on the

command line when you started BBLIST, use the LOAD-BINARY option to select a

file/data set to display/convert.  As used here, a binary data set consists of

one or files using an unbroken sequence of numbers as the file name extension.

After the file loads, BBLIST shows information about this binary data set.

Paragraph 4-3 explains this information.

4-2.2.  Format.  Use the LOAD-FORMAT option to select a format file for load-

ing.  A format file stores all your BBLIST menu settings and conversion layout

selections.

4-3.  INFORMATION.  This screen shows the following information about the

file(s) in the currently loaded binary data set.

4-3.1.  "nnn" Binary File(s).  This box lists the number (nnn), name, exten-

sion, and size of all the files in the currently loaded binary data set.  You

can load any file in this series by moving the cursor to the desired file and

pressing <Enter>.

4-3.2.  Format File Name.  This box shows the name of the file BBLIST is using

to format the binary data.

4-3.3.  Ensemble Info in Data Set.  This box shows the number, date, and time

of both the first and last ensemble in the entire data set.

4-3.4.  Ensemble Info in "filename.ext".  This box shows the number, date, and

time of the first ensemble in the binary data file (filename.ext) highlighted

by the cursor bar.

4-3.5.  Recorded Data.  This box shows the data collected by the BBADCP for

this ensemble and the number of bytes used to record this data (Ensemble

size).  Data from the BBADCP can include:  leader, velocity (in the beam,

instrument, ship, or earth-coordinate system), correlation, intensity,

percent-good, status data, bottom-track, CTD, and vertical-beam data.

4-4.  EXIT <Alt-X>.  Selecting this menu item, or pressing <Alt-X> in any

menu, lets you exit the BBLIST program.  If you changed any menu settings,

BBLIST asks you to save your format settings before exiting the program.

5 - PROCESS MENU

----------------

Use this menu to set the processing parameters for BBLIST.  You can set the

velocity reference, magnetic variation (declination), velocity measurement

units, and depth measurement units.

5-1.  VELOCITY REFERENCE.  Use this option to set the reference source for

BBADCP velocity data.  Press the <Spacebar> to cycle through the options of

ADCP, BTM TRACK, and REF LAYER.

----------------------------------- Page 4 -----------------------------------

* ADCP - This option is typically used when the ADCP is fixed in place on a

non-moving platform (other than roll and pitch motion).  BBLIST does not

alter this "raw" ADCP velocity data when using this option.

* BTM TRACK - This option is typically used when the ADCP is aboard a moving

vessel and the bottom is in tracking range.  This option removes ship's

speed and shows all velocities as true water velocities.  The ADCP must

have collected bottom-track data for this option to be valid.

* REF LAYER - This option is typically used when the ADCP is aboard a moving

vessel, and the bottom is NOT in tracking range or is providing bad data.

This option removes both ship's speed and the reference layer velocity,

showing all velocities as true water velocities.  The ADCP must have col-

lected reference layer data for this option to be valid.

5-2.  MAGNETIC VARIATION.  Use this option to enter the magnetic declination

(heading variance caused by earth's magnetic field) at the deployment site

(-180.00 to +180.00 degrees).  For example, if a map of the deployment site

showed magnetic declination to be 15 degrees east, enter a value of +15 de-

grees.  This entry is typically used for BBADCP's using an internal compass.

5-3.  VELOCITY UNITS.  Use this option to set the scaling units for BBADCP

velocity data.  Press the <Spacebar> to cycle through the options of ADCP

(scaling in same units as collected by the BBADCP), millimeters per second

(mm/s), centimeters per second (cm/s), meters per second (m/s), feet per sec-

ond (ft/s), and knots (kt).

5-4.  DEPTH UNITS.  Use this option to set the scaling units for BBADCP depth

data.  Press the <Spacebar> to cycle through the options of ADCP (scaling in

same units as collected by the BBADCP), centimeters (cm), meters (m), and feet

(ft).  Note that when using ADCP units, some depth data are in decimeters

(dm); 1 decimeter = 0.1 meter; 1 meter = 10 decimeters.

5-5.  MARK BELOW BOTTOM.  Use this option if you want to mark your velocity

data "bad" below the bottom.  If the Bottom Track Range is not valid, data

are not marked "bad."  If velocity data are collected in the beam-coordinate

system, each beam is marked "bad" separately before transformations are done.

For other coordinate systems, data below the minimum bottom-track range are

marked "bad."

6 - DISPLAY MENU

----------------

Use this menu to select the type of data to view.  Data available for display

include ADCP setup, sensors, reference layer, bottom-track, and profile.

6-1.  ADCP SETUP.  This option shows the following information about the

BBADCP hardware, firmware, setup, and sensors available.

6-1.1.  ADCP Hardware and Firmware.  This box shows transducer and firmware

information taken from the BBADCP fixed leader data.

* Frequency - Acoustic frequency of BBADCP in kHz

* Beam angle - Mounting angle of transducer beams in degrees

* Number of beams - Number of profiling beams

* Orientation - Vertical pointing direction (up or down)

* Beam pattern - Transducer mounting design (concave or convex)

* Transducers - Electrical connection to chassis (connected or disconnected)

* CPU firmware - Version detected

----------------------------------- Page 5 -----------------------------------

6-1.2.  Sensors Available and Used.  This box shows whether the following

sensors were available (yes/no), and if they were available, whether they were

used during data collection (yes/no).  BBLIST reads this information from the

BBADCP fixed leader data.

* Depth sensor

* Heading sensor

* Pitch sensor

* Roll sensor

* Salinity sensor

* Temperature sensor

6-1.3.  ADCP Setup.  This box shows the BBADCP setup as read from the fixed

leader data.  A dash (-) in any field means information was not available from

the BBADCP.

* Source of data - ADCP, simulated, or unknown

* Number of bins - WN-command (number of depth cells)

* Bin length - WS-command (depth cell size)

* Blank after transmit - WF-command

* Pings per ensemble - WP-command

* Time per ping - TP-command (time between ping groups)

* Profiling mode - WM-command

* Heading bias - EB-command

* Heading alignment - EA-command

* Low corr. threshold (or min ping correlation) - WC-command

* Number of code repetitions (or max ping correlation)

* % good minimum - WG-command

* Speed of sound source - calculated or fixed (EZ-command)

* Error vel. threshold - WE-command

* Bin map used - Yes/no (only when transformation done in ADCP; EX-command)

* 3-beam solution used - Yes/no (only when transformation done in ADCP;

EX-command)

* Code element lag - Time between transmit codes in counts

* Lag - Lag distance between transmit pulses

* Distance to first bin - Distance to middle of first depth cell

* Transmit length - &X-command

----------------------------------- Page 6 -----------------------------------

6-2.  SENSORS / REF LAYER.  This option shows information from the BBADCP

sensors (internal and external) and information about the reference layer.

6-2.1.  Sensors.  This box shows BBADCP sensor data taken from the variable

leader data.  The BBADCP collected this information with each ping.  An (f)

symbol means the BBADCP used a manually-entered "fixed" value instead of a

sensor reading (see par. 6-1.2).

* BIT result - Built-in test result code (not yet implemented in firmware)

* Heading - Fixed/real heading angle of BBADCP beam 3

* Pitch - Fixed/real BBADCP pitch angle (+ = beam 3 higher than beam 4)

* Roll - Fixed/real BBADCP roll angle (UP-facing systems:  + = beam 2 higher

than beam 1; DOWN-facing systems:  + = beam 1 higher than beam 2)

* ADCP depth - Fixed/real depth of transducer head (used to calculate real

depth of bins)

* Temperature - Fixed/real water temperature in degrees C

* Salinity - Fixed/real water salinity value in parts per thousand

* Speed of sound - Fixed/calculated speed of sound value

* Heading standard deviation - Accuracy of compass heading readings

* Pitch standard deviation - Accuracy of gyrocompass/pendulum pitch readings

* Roll standard deviation - Accuracy of gyrocompass/pendulum roll readings

* Max ping time - Maximum time between ping groups in ensemble

* ADC channels (0-7) - Analog-to-digital convertor channel output (see PT1

and PT2 commands in Appendix-C of BBADCP Technical Manual for scaling)

* Average of pressure sensor

* Pressure sensor standard deviation

6-2.2.  Reference Layer Parameters.  This box shows the reference layer size

as read from the bottom-track data format.

* Min reference layer - BL-command (minimum layer size setting)

* Near reference layer - BL-command (near-boundary setting)

* Far reference layer - BL-command (far-boundary setting)

6-2.3.  Reference Layer Velocities.  This box shows reference layer velocity

data.  Use the PROCESS menu to set VELOCITY UNITS, VELOCITY REFERENCE, and

MAGNETIC VARIATION.  The effects of speed of sound are taken into account.

* Velocity 1/2/3/4 (beam, instr, ship) - Reference layer velocities in the

indicated coordinate system (see Table 1).

* East velocity - East(+)/west(-) reference layer velocity component

* North velocity - North(+)/south(-) reference layer velocity component

* Vertical velocity - Up(+)/down(-) reference layer velocity component

* Error velocity - Data validity check of reference layer velocities

* Horizontal magnitude - Base speed of the reference layer water current

* Direction - Base direction of reference layer water current

Table 1.  Velocity Reference Criteria

----------+---------------+---------------+---------------+---------------

Coord Sys |  Velocity 1   |  Velocity 2   |  Velocity 3   |  Velocity 4

----------+---------------+---------------+---------------+---------------

|            Positive values indicate water movement:

Beam    |  To beam 1    |  To beam 2    |  To beam 3    |  To beam 4

Instr   |  Bm1 to Bm2   |  Bm4 to Bm3   |  To xducer    |  Error vel

Ship    |  Port to Stbd |  Aft to Fwd   |  To surface   |  Error vel

Earth   |  To east      |  To north     |  To surface   |  Error vel

----------+---------------+---------------+---------------+---------------

NOTE:  Coordinate system is set by EX-command in BBADCP.

----------------------------------- Page 7 -----------------------------------

6-2.4.  Reference Layer Beam Data.  This box shows reference layer correla-

tion, echo intensity, and percent-good data.

* Beam 1/2/3/4 Corr - Reference layer correlation data by beam

* Beam 1/2/3/4 Inten - Reference layer echo intensity data by beam

* Beam 1/2/3/4 % Good - Reference layer percent-good data by beam

6-3.  BOTTOM TRACK.  This option shows the following information about bottom-

track data.

6-3.1.  Bottom Track Setup.  This box shows the bottom-track setup parameters

as read from the bottom-track data format.

* Number of pings - BP-command (bottom-track pings per ensemble)

* Delay before reacquire - BD-command

* Min correlation - BC-command (correlation magnitude minimum)

* Min eval amplitude - BA-command

* Min percent good - Not fully defined in firmware; preset to 25

* Max error - BE-command (bottom-track error velocity maximum)

* Mode - BM-command (bottom-track mode)

6-3.2.  Bottom Track Parameters.  This box shows the bottom-track ping parame-

ters as read from the bottom-track data format.

* Number search pings - Number of pings used to search for the bottom

* Number track pings - Number of pings used to track the bottom

* Max tracking depth - BX-command

6-3.3.  Bottom Track Beam Data.  This box shows bottom-track correlation, echo

intensity, and percent-good data.

* Beam 1/2/3/4 Corr - Bottom-track correlation data by beam

* Beam 1/2/3/4 Amp - Bottom-track evaluation amplitude data by beam

* Beam 1/2/3/4 % good - Bottom-track percent-good data by beam

6-3.4.  Bottom Track Velocities.  This box shows bottom-track velocity data.

Use the PROCESS menu to set VELOCITY UNITS, VELOCITY REFERENCE, and MAGNETIC

VARIATION.  The effects of speed of sound are taken into account.

* Velocity 1/2/3/4 (beam, instr, ship) - Bottom-track velocities in the

indicated coordinate system (see Table 1).

* East velocity - East(+)/west(-) bottom-track velocity component

* North velocity - North(+)/south(-) bottom-track velocity component

* Vertical velocity - Up(+)/down(-) bottom-track velocity component

* Error velocity - Data validity check of bottom-track velocities

* Horizontal magnitude - Base speed of the bottom-track water current

* Direction - Base direction of bottom-track water current

6-3.5.  Bottom Track Vertical.  This box shows bottom-track vertical range

and depth data.

* Beam 1/2/3/4 Depth - Bottom-track vertical depth data by beam

* Beam 1/2/3/4 Range - Bottom-track vertical range data by beam

----------------------------------- Page 8 -----------------------------------

6-4.  DISPLAYED BINS.  This option lets you select the bins to view on the

screen.

* Range - Selects a sequential series of bins to display.  With this option,

you define the following.

. First bin - Starting bin number

. Last bin - Ending bin number

. Bins to skip - The number of bins to skip in the sequence.  For exam-

ple, if FIRST BIN = 10, LAST BIN = 20, and BINS TO SKIP = 0, BBLIST

displays bins 10, 11, 12...20.  Setting BINS TO SKIP = 2, displays

bins 10, 13, 16, and 19.

* List - Lets you select a mixed series of bins to display using separators.

For example, if you entered "1-5,10,20-25,7,123", BBLIST displays the fol-

lowing bins in the following order:  1, 2, 3, 4, 5, 10, 20, 21, 22, 23,

24, 25, 7, 123.

6-5.  SELECT DATA ORDER.  This option lets you select the left-to-right dis-

play order of the profile data.  Set the order by entering the highlighted

letter of the desired profile data.  Use a space to separate "page 1" from

"page 2."  For example, the default order (selected by pressing <F4>) is

NVEMR DCIPS.  This selection shows bin Number, Velocities (see Table 1), Earth

velocities, Magnitude and direction, and bin Range on page 1 of the display.

Page 2 of the display shows the Depth profile, Correlation, Intensity,

Percent-good, and Status data.  You can select the same data field more than

once.  Any data NOT selected for page 1 and page 2 is automatically shown on

additional pages.  You can move between pages using the left and right arrow

keys.  You can view other ensembles in the data set by using the <+>, <->, and

<J>ump keys.

BBLIST can display the following data in the sequence chosen by the SELECT

DATA ORDER option.  The displayed profile data corresponds to the bins (depth

cells) selected in the DISPLAYED BINS option.  You can immediately view these

data by selecting the highlighted letter associated with the data (N/R/D/V/E/

M/C/I/P/S/F).  When you select a data item for viewing in this manner, BBLIST

shows the display page that contains the data and "flashes" the column title.

BBLIST shows dashes (-) if the BBADCP did not collect the associated data.

* bin Number - This option shows the bin (depth cell) number for the associ-

ated profile data.

* bin Range - This option shows bin range data.  Bin range is the distance

of the depth cell from the transducer head, NOT the distance below the

surface.  The PROCESS-DEPTH UNITS option sets the unit of measurement.  If

you select ADCP units, data are in centimeters.

* Depth profile - This option shows depth profile data.  Depth data are the

distances of the depth cells below the surface.  BBLIST uses ADCP depth

from the DEPTH SENSOR reading to calculate the depth profile.  The

PROCESS-DEPTH UNITS option sets the unit of measurement.

* Velocities (beam, instr, ship) - This option shows water-track velocities

in the indicated coordinate system (see Table 1).  Use the PROCESS menu to

set VELOCITY UNITS and VELOCITY REFERENCE.  The effects of speed of sound

are taken into account.

----------------------------------- Page 9 -----------------------------------

* Earth velocities - This option shows velocity data referenced in earth

coordinates (east/west, north/south, up/down, error).  East, north, and up

components are positive values.  The horizontally-scaled error component

is the result of the fourth beam data-redundancy check and is usually a

"low" value compared to the other components.  Use the PROCESS menu to set

VELOCITY UNITS, VELOCITY REFERENCE, and MAGNETIC VARIATION.  The effects

of speed of sound are taken into account.

* Magnitude and dir - This option shows the base horizontal magnitude

(speed) and direction of the water-current data.  Use the PROCESS menu to

set VELOCITY UNITS, VELOCITY REFERENCE, and MAGNETIC VARIATION.  The ef-

fects of speed of sound are taken into account.

* Correlation - This option shows correlation data.  Measurement units are

in BBADCP "counts."  Correlation data give the magnitude of the normalized

echo autocorrelation at the lag used for estimating the Doppler phase

change.  The BBADCP represents this magnitude by a linear scale between 0

and 255, where 255 is perfect correlation (i.e., a solid target).

* Intensity - This option shows echo intensity data.  Echo intensity indi-

cates the strength of the return signal.  Measurement units are in BBADCP

"counts."  Each count is about 0.45 dB.

* Percent good - This option shows percent-good data.  Percent-good is a

data-quality indicator that reports the percentage (0-100) of "good" data

collected for each depth cell in the velocity profile.  The exact meaning

of the percent-good data depends on the velocity coordinate system in

which you collected your data (set by EX-command).  See Appendix D of the

BBADCP Technical Manual for a complete explanation.

* Status - This option shows status data.  These data contain information

about the status and quality of BBADCP data.  A value of "0" means the

measurement was good.  A value of "1" means the measurement was bad.

* Vertical (fifth) beam - For vertical-beam systems, this option shows

velocity, correlation, intensity, percent-good, and status data for the

vertical/fifth beam.

6-6.  CTD/VERT. BEAM.  For vertical-beam systems using CTD, this option shows

the following information about CTD data and the vertical-beam setup.

6-2.1.  CTD Data.  This box shows sensor data taken from the CTD/OBS

interface.

* CTD/OBS hardware type - Type of hardware

* Data flag - 0 = data not updated, 1 = new data

* Temperature - CTD temperature data in counts

* Conductivity - CTD conductivity data in counts

* OBS voltage - Observed backscatter voltage data in counts

* Pressure - CTD submergence/depth/pressure data in counts

6-2.2.  Vertical-Beam Setup.  This box shows the vertical-beam commands.

* Number of beams      (VN-command)    * Pings per ensemble (VP-command)

* Depth cell length    (VS-command)    * Profiling mode     (VM-command)

* Blank after transmit (VF-command)

---------------------------------- Page 10 -----------------------------------

7 - CONVERT MENU

----------------

Use this menu to convert the binary BBADCP data set into ASCII.  Before start-

ing the conversion process, you can define the conversion limitations and the

output format.

7-1.  LIMITS.  This option lets you define several conversion limitations and

parameters.  Your selections limit (or expand) the amount of data converted.

7-1.1.  Bin Selection.  This option lets you select the bins (depth cells)

that BBLIST will convert to ASCII.

* Range - Selects a sequential series of bins to convert.  With this option,

you define the following.

. First bin - Starting bin number

. Last bin - Ending bin number

. Bins to skip - The number of bins to skip in the sequence.  For exam-

ple, if FIRST BIN = 10, LAST BIN = 20, and BINS TO SKIP = 0, BBLIST

converts data for bins 10, 11, 12...20.  Setting BINS TO SKIP = 2,

converts data for bins 10, 13, 16, and 19.

* List - Lets you select a mixed series of bins to convert using separators.

For example, if you entered "1-5,10,20-25,7,123", BBLIST converts data for

the following bins in the following order:  1, 2, 3, 4, 5, 10, 20, 21, 22,

23, 24, 25, 7, 123.

7-1.2.  Add Data to Missing Bins.  This option tells BBLIST to "pad" missing

bin data with zeros or with an unrealistic value (see Table 2).  For example,

the BBADCP collected data for 100 bins, but your external user program always

expects to see data for 128 bins.  Setting this option to YES, fills the re-

maining 28 bins with false data.

7-1.3.  Ensemble Increment.  This option defines the number of ensembles to

increment during the conversion process.  For example, if set to "1", BBLIST

processes every ensemble (within the limits defined by the PRUNING METHOD).

If ENSEMBLE INCREMENT = 2, BBLIST processes every other ensemble.

Table 2.  Padding Values for Unrealistic/Bad Data

-------------+--------------------------------

Data     |      Unit of measurement:

types    | mm/s  | cm/s | m/s | ft/s | kt

-------------+-------+------+-----+------+----

Velocity     | 99999 | 9999 | 99  |  99  | 99

Magnitude    | 99999 | 9999 | 99  |  99  | 99

-------------+-------+------+-----+------+----

NOTE:  BBLIST sets all other data fields (corre-

lation, intensity, percent-good, etc) to zero.

---------------------------------- Page 11 -----------------------------------

7-1.4.  Pruning Method.  This option lets you select the range of ensembles to

process based on either the ensemble time or the ensemble number.

* Ensemble number - Selects sequential ensembles to convert using a starting

and ending ensemble number.  With this option, you define the following.

. First ensemble - Starting ensemble number (FIRST IN SET shows the low-

est ensemble number in the data set.)

. Last ensemble - Ending ensemble number (LAST IN SET shows the highest

ensemble number in the data set.)

* Recording time - Selects sequential ensembles to convert using a starting

and ending date/time.  With this option, you define the following.

. Start date - Starting date (START DATE IN SET shows the earliest re-

cording date in the data set.)

. sTart time - Starting time (START TIME IN SET shows the earliest re-

cording time in the data set for the selected start date.)

. End date - Ending date (END DATE IN SET shows the latest recording

date in the data set.)

. eNd time - Ending time (END TIME IN SET shows the latest recording

time in the data set for the selected end date.)

7-1.5.  Write Missing Data from ADCP.  This option tells BBLIST to "pad" miss-

ing bin data with zeros or with an unrealistic value (see Table 2).  For exam-

ple, the BBADCP did not collect percent-good or status data, but your external

user program always expects to see data for these fields.  Setting this option

to YES, fills the missing fields with false data.

7-1.6.  Go Auto to Next File in Data Set.  This option lets you choose to load

the next file in the data set automatically. When set to YES, BBLIST loads the

next file in the data set based on the extension (000, 001, 002, ...).  If the

data set is missing a file in the sequence, BBLIST stops processing.  If the

file does not have a numeric extension, BBLIST only processes that file.

7-1.7.  Max File Size.  The option lets you set the maximum size of the ASCII

output data file.  The actual size of the output file may be slightly less

than your entry.  This is because BBLIST writes only complete ensembles to the

ASCII files.  No ensemble is split between two output files.  If the file size

is set to zero, the maximum file size is unlimited.  If you plan to transfer

data to DS-DD floppy disks (360KB), we recommend a maximum file size of 300KB.

7-2.  DEFINE FORMAT.  This option defines the format and data fields used dur-

ing the ASCII file conversion.  When you first enter this screen, BBLIST dis-

plays only a FORMAT END marker.  To begin entering data fields, press <Enter>

to display the DEFINE FORMAT selection menu.  Before describing the menu op-

tions available in the DEFINE FORMAT menu, we will give a brief overview of

the display screen.

As you begin to enter data fields, notice that the screen shows the location

of the field in a highlighted block.  Any editing actions affect the entire

highlighted block.  Notice when you have profile and non-profile data in the

same row, the non-profile data expands to match the length of the profile

data.  For example, in the vertical layout mode, if you have 10 bins of veloc-

ity profile data starting on line 1, and you also put the ensemble number

(non-profile data) on line 1, the ensemble number expands to cover 10 rows.

---------------------------------- Page 12 -----------------------------------

To see data as it will appear in the output file, either look at the layout at

the bottom of the screen for a display of one row, or press <F9> to see the

entire data set.  Use <F5>/<F6> to remove/add spaces at the left side of a

conversion field.  Use <F7>/<F8> to remove/add spaces or decimal places at the

right side of a conversion field.  When using <F9>, you can see how your ASCII

output data will look using the <Tab>/<Shift-Tab> (horizontal scroll), <Home>/

<End>, <PgUp>/<PgDn>, <+>/<->, and <J>ump keys.

The bottom part of the DEFINE FORMAT screen shows the following information

that should help you design your layout.

* Line - Shows information about the line highlighted by the cursor block.

. No. - The line number of the first item in the cursor block

. Total length - Shows the number of characters used by all the fields

in the current line.  This number is helpful for determining printer

and program line length limitations.  If line length is more than 1024

characters, try to change your layout to reduce the length.

* Field - Shows information about the field highlighted by the cursor block.

. No. - The field (column) number of the item in the cursor block

. Len - Shows the character length of the cursor block.  The F5, F6, F7,

and F8 sizing keys affect this value.  The maximum field length is 20

characters (255 characters for user comment lines).  Even though you

can set the length to a value smaller than the actual length of the

field, BBLIST will NOT truncate the value of the field.  Instead,

BBLIST just removes all EXTRA spaces.  Use a length of "1" when you

want to optimize space (compress the data) in the ASCII file.  The

following example shows what the data would look like for different

field length.

------+---------------

Field | Field length:

value |   5   |   1

------+-------+-------

-12   | xx-12 | -12

12    | xxx12 | 12

12345 | 12345 | 12345

1     | xxxx1 | 1

------+-------+-------

x = space

. Dec - Shows the number of decimal places used in the cursor block.

The maximum decimal length is 6 places.  The F7 and F8 sizing keys

affect this value.

* Field description line (line 21) - Shows detailed information about the

field highlighted by the cursor block.  This information can include field

name, unit of measurement, and velocity reference.

* Numeric field layout (e.g., "XXX.XX") - Shows the format of the field as

set by the sizing keys.

* Data from "filename.ext" (line 22) - Shows the filename from which BBLIST

is extracting the binary data used in the conversion process.

---------------------------------- Page 13 -----------------------------------

* Line layout (line 23) - Shows one line of converted data as it will appear

in the ASCII file.  Note the following special characters.

. Tab - ASCII character number 9

. Carriage return - ASCII character number 13

. Line feed - ASCII character number 10

* Ruler (line 24) - Helps you set the spacing of your fields.

The following options are available in the DEFINE FORMAT menu.

7-2.1.  Bin Layout.  This option defines the layout of the conversion table

using the profile's bin data as the primary sorting method.

* Vertical - Displays bin data in a top-to-bottom format

* Horizontal - Displays bin data in a left-to-right format

If you have entered at least one field in the table, and then select this

option to change the layout method, BBLIST will ask you to verify your deci-

sion.  This is because BBLIST will delete the entire table, forcing you to re-

enter all data fields.  You may want to save your current settings with <F2>

before changing the layout method.

7-2.2.  Ensemble Info.  This option lets you insert information about the

ensemble being processed.  Entry fields include the following.

* Number - Current ensemble number

* Date str (YY/MM/DD) - Date ensemble began recording in a year, month, day

string format (e.g., "92/09/12" = September 12, 1992)

* Year (YY) - Year ensemble began recording (e.g., "92" = 1992)

* Month (MM) - Month ensemble began recording (e.g., "09" = September)

* dAy (DD) - Day of the month ensemble began recording

* Time str (HH:MM:SS) - Military time ensemble began recording in an hour,

minute, second string format (e.g., "14:23:02")

* tIme str (HH:MM:SS.CC) - Military time ensemble began recording in an

hour, minute, second, hundredths of seconds (centiseconds) string format

(e.g., "14:23:02.35")

* Hour (HH) - Hour (24-hour clock) ensemble began recording

* minUtes (MM) - Minute ensemble began recording

* Seconds (SS) - Second ensemble began recording

* Centiseconds (CC) - Hundredths of seconds ensemble began recording

* timE of day [s] - Time of day in seconds ensemble began recording (e.g.,

"51782" = 14:23:02)

* time oF day [cs] - Time of day in hundredths of seconds ensemble began

recording (e.g., "5178235" = 14:23:02.35)

* decimaL day - Sequential day of the year ensemble began recording in deci-

mal format, where the decimal portion represents the time of day as a

percentage (e.g., "252.25" = September 10 [in a non-leap year] at 06:00;

"253.75" = September 10 [in a leap year] at 18:00)

---------------------------------- Page 14 -----------------------------------

7-2.3.  Sensors.  This option lets you insert information about internal/ex-

ternal sensors.  Entry fields include the following (see par. 6-2.1).

* Built in test                       * spEed of sound

* Heading                             * headinG std dev

* Pitch                               * pitCh std dev

* Roll                                * roLl std dev

* ADCP depth                          * Max ping (delta t)

* Temperature                         * adc chaNnels

* Salinity                            * ctD/obs data

* pressUre                            * pressure std deV

7-2.4.  REFERENCE LAYER.  This option lets you insert information about the

reference layer.  Entry fields include the following (see listed paragraphs

for a description of the data).

* Velocity - Reference layer velocity data in relation to the ADCP beams;

ALL, or in the indicated coordinate system (par. 6-2.3 and Table 1)

* Earth vel - Reference layer velocity data expressed in earth coordinates;

ALL, or East, North, Vertical, and ERror components (par. 6-2.3)

* Mag and dir - Reference layer velocity data expressed as horizontal magni-

tude (speed) and direction; ALL, or Magnitude and Direction components

(par. 6-2.3)

* Correlation - Reference layer correlation data; ALL, or individual beams

(par. 6-2.4)

* Intensity - Reference layer echo intensity data; ALL, or individual beams

(par. 6-2.4)

* Percent good - Reference layer percent-good data; ALL, or individual beams

(par. 6-2.4)

* Ref layer - Reference layer size parameters; ALL, or Near and Far compo-

nents (par. 6-2.2)

7-2.5.  Bottom Track.  This option lets you insert bottom-track data.  Entry

fields include (see listed paragraphs for a description of the data):

* Velocity - Bottom-track velocity data in relation to the ADCP beams; ALL,

or in the indicated coordinate system (par. 6-3.4 and Table 1)

* Earth vel - Bottom-track velocity data expressed in earth coordinates;

ALL, or East, North, Vertical, and ERror components (par. 6-3.4)

* Mag and dir - Bottom-track velocity data expressed as horizontal magnitude

(speed) and direction; ALL, or Magnitude and Direction components (par.

6-3.4)

* Range - Bottom-track range data in relation to the ADCP beams; ALL, or

individual beams (par. 6-3.5)

* Correlation - Bottom-track correlation data; ALL, or individual beams

(par. 6-3.3)

* Amplitude - Bottom-track evaluation amplitude data; ALL, or individual

beams (par. 6-3.3)

* Percent good - Bottom-track percent-good data; ALL, or individual beams

(par. 6-3.3)

* variaBles - Bottom-track setup parameters; ALL, or the number of Search

pings, number of Track pings, and maximum tracking Depth (par. 6-3.2)

* Depth - Bottom-track depth data in relation to the ADCP beams; ALL, or

individual beams (par. 6-3.5)

---------------------------------- Page 15 -----------------------------------

7-2.6.  Profiles.  This option lets you insert water-track profile data.

Entry fields include (see par. 6-5 for a description of the data):

* Velocity - Water-track velocity data in relation to the ADCP beams; ALL,

or in the indicated coordinate system (Table 1).  Vertical (fifth) beam

data should be selected separately.

* Earth vel - Profile velocity data expressed in earth coordinates; ALL, or

East, North, Vertical, and ERror components

* Mag and dir - Profile velocity data expressed as horizontal magnitude

(speed) and direction; ALL, or Magnitude and Direction components

* Correlation - Profile correlation data; ALL, or individual beams

* Intensity - Profile echo intensity data; ALL, or individual beams

* Percent good - Profile percent-good data; ALL, or individual beams

* Status - Profile status data; ALL, or individual beams

* Depth ref - Profile depth data expressed as a Bin number, a Range from the

transducer head, or a Depth from the surface

* Vert. beam - Profile data for the vertical (fifth) beam expressed as

Velocity, Correlation, Intensity, Percent good, Status, Bin number, Range

from the transducer head, or Depth from the surface.

7-2.7.  User Comment.  This option lets you insert notes (up to 255 characters

including carriage return and line feed).

7-2.8.  Insert Line.  This option lets you insert a blank row.  You can then

insert data fields into this new row.

7-2.9.  Clear Table.  This option lets you delete the entire format layout.

BBLIST first asks you to verify your decision because of the destructive na-

ture of this action.  If you accidentally clear the table, just reload your

last used format file.  This action assumes you recently saved the format.

7-2.10.  Delete Block.  This option lets you delete the highlighted field(s).

Be careful with this option because BBLIST does NOT ask you to verify your

decision.  If you accidentally delete a block, you will have to re-enter it

manually or reload your last used format file (assuming you recently saved the

format).

7-2.11.  Field Delimiter.  This option defines how to separate each data field

in the row.  The delimiter used may be a requirement of any external program

to which you are sending the ASCII data.  Options include:

* Space - Used for easiest viewing

* Comma - Used by some data base programs

* Tab - On screen, BBLIST displays the ASCII tab character (decimal charac-

ter number 9).

---------------------------------- Page 16 -----------------------------------

7-3.  START CONVERSION.  This option starts the conversion process.  Enter a

file name for the ASCII file data set.  BE SURE TO USE A FILE NAME THAT DIF-

FERS FROM ANY EXISTING FILE NAME.  DO NOT USE THE NAME OF THE BINARY DATA SET.

IF BBLIST DETECTS THAT THE FILE NAME ALREADY EXISTS, YOU ARE GIVEN THE OPTION

TO OVERWRITE THE EXISTING FILES.  If you do NOT enter an extension, BBLIST

automatically generates a sequential extension for each file created beginning

with ".000".  If you enter a numeric extension, BBLIST will use that extension

as the first one in the converted ASCII data set.  If you enter a non-numeric

extension, BBLIST creates only one file (up to MAX FILE SIZE in length), and

then stops the conversion process.  After entering the file name, press <En-

ter> to start the process.  You can stop the conversion process by pressing

<Esc>.  BBLIST will finish converting the current ensemble before it stops the

process.  If the binary data set has a missing file in the sequence, BBLIST

will stop processing.  If the binary data file does not have a numeric exten-

sion, BBLIST will only process the selected file.

NOTE:  To speed up the conversion process, you may want to create the ASCII

file on a RAM drive (see DOS manual for details).  The next best option is to

create the file on a hard drive.  The slowest method is to create the file on

a floppy drive.

7-4.  REPORT FILE.  When the conversion process is complete, BBLIST creates an

ASCII report file (*.RPT).  You can view this file with any text editor.  This

file contains the following information about the settings and data in the

converted files.

* ADCP information - system frequency, beam angle, number of profiling

beams, transducer orientation (up/down), transducer pattern (concave/con-

vex), transducer connection (connected/disconnected), and CPU firmware

version number

* ADCP setup - number of bins, bin length, blank after transmit length,

pings per ensemble, time per ping, and profiling mode.

* ASCII file data format - a description on the contents of each line in the

converted data file (for one ensemble).

* Processing parameters - velocity units, velocity reference, depth units,

bin sequence, magnetic variation.

7-5.  BBBATCH PROGRAM.  When you have mastered the BBLIST program, you can use

BBBATCH.EXE to convert binary data sets to ASCII data sets in a DOS batch

mode.  This comes in handy when you have several data sets to convert or have

a large data set that you want to convert overnight.  Here is the syntax for

BBBATCH.

BBBATCH BinaryFileName FormatFileName AsciiFileName

BinaryFileName = name of the binary data set to convert (no extension needed)

FormatFileName = name of the format file to use

AsciiFileName  = name of the ASCII data set (unique name)

Running BBBATCH without any command line parameters displays the syntax.

-- END OF TEXT --