***It is suggested that the scientists do their own processing of the raw data for their specific purposes.***

# The data have been preliminarily processed by the ship’s technician during the cruise for the convenience of an initial sensor check. These files should not be taken as the final processed product for analysis.

EN448 CTD PROCESSING:

Instructions for processing are in the Sea-Bird files EN448\CTD\  manualSBE4249.txt & SBEDataProcessing_5.37.pdf

Raw data is in EN448\CTD\sswin\data

Processed data by Endeavor Technician Lynne Butler is in EN448\CTD\sswin\data\list

Processed data by WHOI Engineer Olga Kosnyreva is in EN448\CTD\CTDprocessed

# All ctd###.*  casts were processed for down cast only, at 0.1m pressure bins, with the ctd###.asc file showing pressure and depth as well as numerous other parameters.

See CTDSysCon448.doc and other files in EN448\CTD\  for documentation of sensor serial numbers, programs, processing, and equipment used.

Original Con files were as follows:

Casts CTD001-067.*  used  448_0607_12plb.CON

CTD on deck transmissometer, altimeter, PAR, DO1, DO2, Fluorometer, Surface PAR & pressure tests are in:

EN448\CTD\sswin\CalData\  448xapooFs12_0607.dat & 448xapooFs13_0607_24pl.dat

For pump casts: Install male end of pump hose on outside of frame stanchion level with Temp & Cond sensor end of CTD. Lower PAR sensor so it doesn’t get damaged by hose.

For details of problems with pumps, See Jefferson Turner &/or Dennis McGillicuddy.

EN448 CTD cast notes:

winch 1 speeds: 30m/min max 0-100m, 60m/min max 100+m.

Start w/12ea 10L GO Niskins, internal monofilament & spring tension. Run lanyards to left of bottle to prevent interference with high volume valves. Lanyards also fit on OTE Ext Tension Niskins.

Lanyard loops on center lever release mechanism not used.

Carousel electronics & lever release mechanism has 24 positions. Fire every other position in Table driven bottle firing setup in sswin.

--------------------------------------------------------------------------

CTD gets GPS from bridge switch, whatever they have selected is the unit that is used. 4800 baud.

NMEA error in sswin. Check to see if, on the deck unit, green NMEA light is off. Red error light may also be flashing or on.

Restart computer, log out, cycle power on deck unit, problem may go away. Check GPS cables. Use linemon or hyperterminal to check NMEA signal from splitter box.

--------------------------------------------------------------------------

CTD001-006 SPAR not installed. Overcast skies, first day of cruise. SPAR was installed on all other casts (CTD007-067).

See paper Log sheets for logs of Niskin mishaps & misfires and CTD frame changes. Frame was changed from 12 pl to 24 pl second day of cruise. Carousel was changed mid-cruise.

CTD067 Last cast of EN448.  