ADCP -		Acoustic Doppler Current Profiler. Endeavor has both 75 & 300

KHz units. Only the 75 was used during EN451.

CTD –			Conductivity Temperature Depth. Generally refers to the entire

CTD profiling package which also has third party sensors.

CTDprocessed – CTD data processed by Olga Kosnyreva

ssfiles - 	CalibrationSheets451, Application Notes SBE, ECOfluor

sspro - 	SBEDataProcessing-Win32 and processing configuration files

sswin – 	CalData  451  on deck calibration and test cast files

data – raw data

list – CTD data processed by Lynne Butler w/out LoopEdit

014 – original files which were incorrectly named 0014

Documents -		Various files some general, pertaining to the ship, and others

which are cruise-specific. DVD label, Postings, Names=cruise

participants, heights05.doc=antenna & various eqpt locations.

Nobeltec -		EN451 waypoints provided by Dennis McGillicuddy for the

Nobeltec Navigation program.

Photos -		Photos taken during the cruise by participants.

SCS -			Scientific Computing System is NOAA’s program which the

Endeavor uses to compile Navigation, Meteorological, depth,

and other data from the ship’s permanently mounted scientific

instruments. See files in SCS directory for explanation of

sensor strings and Standard Cruise Event header.

Raw -		Sensor strings directly from instruments.

Event -	Various NavMet data saved at 1, 60 & 600 seconds.

TSal -		SBE21, in Wet Lab. Remote temp (T2) is at the seachest, under

the engine room where the underway seawater comes into the

ship. This seawater runs from the seachest through PVC piping

to the Special Purpose Lab, Wet Lab and the 01 deck. Software

to load Sea-Bird Seasave and Processing programs onto a computer are included.

CProgramFilesSea-BirdSBEDataProcessing-Win32

CProgramFilesSea-BirdSeasaveV7

data – raw data

list – CTD data processed by Lynne Butler

Units -		Useful unit conversion program

Notes:

TSG  = SBE45, in Wet Lab.

SST1 = temperature sensor attached to hull at ~1m depth.

SST5 = thermistor sticking down from bottom of ship in transducer well at ~5m depth.