# Get correct bottom depth from Knudsen HF

On CTD computer:

#### Seasave for Windows

If needed: File, Open Configuration, 476_d.psa

Realtime Data, Start

Select Output Data File Name change cast # to next

be sure Begin archiving data immediately is checked or data will not get saved

Start

Enter relevant data into header

Do not press OK yet.

# --------------------------------------------------------------------------------------

##### CTD deployed to 5meters by deck crew

Power ON as soon as CTD is all the way in the water

Pump ON 0011

“Doghouse, Lab, Bring CTD to surface”

OK in header at 3mwo, coming up

#### Check & Log Pressure, Temps, Sals, etc

Begin cast “Doghouse, Lab, Go down to x meters”        where x=requested depth

Closely watch the altimeter, winch wire out, CTD pressure, and echosounder values

“Doghouse, Lab, 10m to go.”

### ***!!!DO NOT TOUCH BOTTOM WITH THE PACKAGE!!!***

# --------------------------------------------------------------------------------------

Upcast, trip bottles

For each bottle trip:  Fire the bottle & make a checkmark on the logsheet

“Doghouse, Lab, Up to the surface”

After last bottle is fired Realtime Data, Stop

#### Power OFF

#### “Doghouse, Lab, CTD power is OFF”

##### CTD recovery by deck crew

# --------------------------------------------------------------------------------------

To print: 	activate & right click desired overlay plot	  Print, OK

At DOS prompt	  D:\procontrol\pro476 CTD###   to process & archive data