EchosounderReadMe476.doc

# R/V Endeavor, EN476, McGillicuddy

# Knudsen 320B/R Echosounder

High Frequency = 12 KHz was not used.

----------------------------------------------------------------------

# Knudsen 3260 Chirp Echosounder

High Frequency = 12 KHz

Low  Frequency = 3.5KHz was not used.

12KHz used to track bottom during entire cruise.

Data saved only on SCS.

Depth data routed to and saved on SCS from beginning until end of cruise.

Navigation input is from the Furuno GP-90D DGPS/WAAS

Sound velocity of 1500m/sec used for entire cruise.

----------------------------------------------------------------------

Digital Depth value in SCS is sometimes not correct if the echosounder is not “tracking” the bottom. Sometimes the echosounder tracks the hard deeper layer rather than the overlaying sediment layer. In those cases the depth value is deeper than actual depth.

The ship’s Echosounder was watched and settings controlled and changed by the CTD computer operator on watch and/or the ship’s tech.

----------------------------------------------------------------------

Transducers are in a transducer well at 5m (bottom of ship’s hull).