SBE Processing configurations used for EN437 Dennis McGillicuddy, Anderson:

# CTD001 through CTD187 have Pressure, dual temperature and conductivity sensors and pumps, WET Labs C-Star 25cm Transmissometer, Benthos Altimeter, SBE43 Oxygen 1, Oxygen 2,

# WET Labs ECO Fluorometer, Biospherical PAR and Surface PAR sensors and 12 Niskin bottles using the smaller frame.

# CTD188 through CTD211 have Pressure, dual temperature and conductivity sensors and pumps, WET Labs C-Star 25cm Transmissometer, Benthos Altimeter, SBE43 Oxygen 1, Oxygen 2,

# WET Labs ECO Fluorometer, Biospherical PAR and Surface PAR sensors and 24 Niskin bottles using the larger frame. All sensors are the same as used on the smaller frame.

# 437Datcnv_a

# Instrument configuration file	C:\sswin\data\ctd001.com

X Match instrument configuration file to input file

# Input directory	C:\sswin\data

# Output directory	C:\sswin\data\list

X Process scans to end of file. Scans to skip over 0. Binary output, Up& downcast, Create both data and bottle file. Scans marked with bottle confirm bit.. Scan range offset (s) 0. Scan range duration (s) 2.

X Merge separate header file.

Scan Count, Time Elapsed (s), Pressure Digiquartz (db), Depth (saltwater, m) Latitude 43.50 used for all casts (should be reprocessed for exact lat for each cast to be accurate), Temp 1 (ITS-90, deg C), Temp 2 (ITS-90, deg C), Temp Diff (ITS-90, deg C), Cond 1 (S/m), Cond 2 (S/m), Cond Diff (S/m), V0, Beam Attenuation Wetlab, Beam Transmission Wetlab, V2, Altimeter (m), V3, PAR, V4, Oxygen V SBE43 1, V5, Oxygen V SBE43 2, V7, Fluorescence WETLab ECOFL (mg/m^3), SPAR, Pump Status, Latitude (deg), Longitude (deg).

437Filter_a

“FILTER can be run before CELLTM to remove any residual response time between the temperature and conductivity sensors and to minimize digitization noise.  On average, a low pass filter with a time constant of 0.03 sec reduces the noise in computed salinity slightly.”

for SBE9/11 SBE9plus, older manualSBE4249.txt suggests

Low pass filter B = 0.15s for pressure

Low pass filter A = 0.03s for conductivity

New SBEDataProcessing_5.32a.pdf suggests - for conductivity and 0.15s for Pressure

# Used: B=0.15 for Press & Depth.

# Not Used as it was in previous years: A=0.03 for Cond 1, 2 & Diff.

# 437AlignCTD_a

# Cond		0.073s (secondary only)

# Temp	0s

# DO		4sec (2 at 25C, 5 at 0C),  V4, Oxy V pri, V5, OxyV

437CellTM_a for 9+ w/TC duct & pump running at 3000rpm.

Correct both pri & sec conductivity values

alpha = 0.03

1/beta = 7

# LoopEdit (not done for EN437)

Fixed minimum velocity of 0.25m/s. X Exclude scans marked bad.

SBE Processing configurations used for EN437 Dennis McGillicuddy, Anderson, continued

# 437Derive_a

Depth (sw, m 43.50 Deg N  used for all casts (should be reprocessed for exact lat for each cast to be accurate. This is an approx average of the starting Latitudes of all casts. It should be checked and entered by scientific personnel), Salinity 1 & 2 (PSU), Density 1 & 2 (sigma-t, Kg/m^3), Oxygen 1 & 2 (ml/l), Pot Temp 1 & 2 (ITS-90, deg C), Sound Velocity 1 & 2 (Chen-Millero, m/s), Descent Rate (m/s)

# 427BinAvg_a

Pressure, 0.1, X Incl #scans/bin, X exclude scans marked bad, skip over 0, Process Downcast only, X Incl surface bin 0, 0, 0

# 427BottleSum_a

X Output min/max values for averaged variables, All Averaged Variables, Derived Variables: Salinity 1 & 2 (PSU), Density 1 & 2 (sigma-t, Kg/m^3), Oxygen Saturation (ml/l), Oxygen 1 & 2 (ml/l), Pot Temp 1 & 2 (ITS-90, deg C), Sound Velocity 1 & 2 (Chen-Millero, m/s).

# 427ASCII_Out_a

# Output header & data files, 60 lines/pg, Label columns top of file, Semicolon, Julian days, All Output variables. Not done: Exclude scans marked bad, Convert elapsed and system time to mm/dd/year hh:mm:ssm Add first column, Replace bad flad.