VERSIONS.DOC

This file describes bug-fixes, modifications, and new features for each

version of SEASOFT

NOTES ON SEASOFT VERSION 4.001

==============================

1. BUG FIXES:

DATCNV, SEASAVE:

---------------

- Incorrect external voltages for SBE16 and SBE21.

2. MODIFICATIONS:

SEACON:

------

- Add offsets to all types of fluorometer sensors.

SEAPLOT, SEASAVE:

----------------

- Plot fluorometer values in log scale.

NOTES ON SEASOFT VERSION 4.002

==============================

1. BUG FIXES:

DATCNV, SEASAVE:

---------------

- Incorrect pressure temperature-compensation values and modulo values for

SBE16 with odd number of external voltages.

- Cannot match pressure sensor serial numbers.

- Cannot find temperature sensor serial numbers in some older .hex flies.

TERM1621:

--------

- Cannot find data when upload.

NOTES ON SEASAVE VERSION 4.003

==============================

1. BUG FIXES:

BINAVG, DATCNV, DERIVE:

----------------------

- Batch job terminates after about 9 files.

CELLTM, FILTER, WILDEDIT:

------------------------

- Output file names are always the same as the input file names.

CFGTOCON, SEASAVE:

-----------------

- Remote-Pressure Display doesn't have its own set of min and max pressure values

to compare to for turning on alarm.

STRIP:

-----

- When running STRIP in batch mode with option -a, it sends all data to the first

.cnv file.

DERIVE:

------

- Cannot derive salinity.

2. NEW FEATURES:

ASCIIOUT:

--------

- Add command line option -f to suppress outputing the flag column.

NOTES ON SEASOFT VERSION 4.004

==============================

1. NEW FEATURES:

- Add an new fluorometer type IFREMER for SBE911 and SBE911Plus.

NOTES ON SEASOFT VERSION 4.005

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- Bad values are not correctly formatted for some of the variables.

DATCNV:

------

- It crashes when .CON files are for SBE16 and created by CFGTOCON.

- Conversion (between metric and english units) problems for temperature

and depth values.

DATCNV, SEASAVE:

---------------

- Conversion (between metric and english units) problems for conductivity,

salinity and pressure.

TRANS:

-----

- Cannot get the bad-value flag from an input file's header.

2. MODIFICATIONS:

BINAVG:

------

- Modify the basic logic.

DATCNV:

------

- Remove the subform for getting latitude for variable depth with fresh water.

TERM1621:

--------

- Remove <F8> from the active key list.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add or modify command line options -b, -bx and -px.

ASCIIOUT:

--------

- Add or modify command line options -dx, -hx, -l, -lx, -f, and -fx.

BINAVG:

------

- Add number-of-scans-to-skipover to the main form.

- Add surface bin parameters to the main form.

#

- Add command line options -bx, -d, -r -rx, -sx, -u, -x, -y, and -z.

DATCNV:

------

- Add salt-water depth into depth variable menu.

- Add number-of-scans-to-skipover to the main form.

- Add command line options -n and -x.

SEAPLOT:

-------

- Add or modify command line options -bvx, -bsx, and -l.

SEASAVE:

-------

- Add or modify command line options -gx, -f and -l.

SPLIT:

-----

- Add down-cast-only to main form.

- Add command line options -d, -dx, -nd, -od<filename>, and -ou<filename>.

NOTES ON SEASOFT VERSION 4.006

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- Programs blow up occasionally.

2. MODIFICATIONS:

DATCNV, SEACON, SEASAVE:

-----------------------

- Delete A/D range and number of frequencies from the SBE911Plus form.

- Set A/D range to 0-5 volts and set number of frequencies to 5 always.

- Delete SBE917 instrument type from SEACON.

- Rename SBE11X to SBE31.

- Get firmware version number for SBE917 from the header of .hex files

rather than being configured in SEACON.

3. NEW FEATURES:

DATCNV, SEACON, SEASAVE:

-----------------------

- Add an new instrument type SBE911e.

NOTES ON SEASOFT VERSION 4.007

==============================

1. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add variable altimeter to sensor list for all types of instrument.

NOTES ON SEASOFT VERSION 4.008

==============================

1. BUG FIXES:

ALIGNCTD:

--------

- Command line option -2 doesn't work.

DATCNV:

------

- If user enters string "cast" in the header part of the input

data file, the program will return "Bad Header" error message.

SEASAVE:

-------

- Command line option -i doesn't work.

2. MODIFICATIONS:

DATCNV, SEACON, SEASAVE:

-----------------------

- Modify strain gauge pressure computation for SBE16 with firmware version >= 4.0.

DERIVE:

------

- Delete command line options -st and -sc.

- Add more variable dependency checking.

SEACON:

------

- Modify the calibration coefficient form for Ph.

3. NEW FEATURES:

DATCNV, SEASAVE:

---------------

- Add an new instrument type SBE17Plus.

DERIVE:

------

- Add an new subform to the main form for input-variable list.

SEAPLOT:

-------

- Add an new output choice to send the displays to files.

NOTES ON SEASOFT VERSION 4.009

==============================

1. BUG FIXES:

ROSSUM:

------

- Cannot handle more than 8 variables in any .CNV files.

NOTES ON SEASOFT VERSION 4.010

==============================

1. BUG FIXES:

LOOPEDIT:

--------

- Cannot handle scans when min velocity was zero.

2. MODIFICATIONS:

DATCNV, SEASAVE:

---------------

- Change units for altimeter to meters|feet.

TERM17:

------

- Set COMM port databits to 8.

3. NEW FEATURES:

SEASAVE:

-------

- Add Ctrl_F2 and Ctrl_F4 to turn off|on control for SBE911Plus and SBE911e.

- Add command line option -o<outFileName> for realtime display in batch mode.

NOTES ON SEASOFT VERSION 4.011

==============================

1. BUG FIXES:

BINAVG, DERIVE:

--------------

- In batch mode some files are processed twice.

CELLTM:

------

- Incorrectly handles secondary sensors.

DATCNV:

------

- Incorrect sound velocity names.

SEASAVE:

-------

- The "EXIT SEASAVE" message gets corrupted when it is in realtime mode with RS232C

connection and the total number of words exceed 13.

- Incorrect values for dz/dt, accel, dox/dt, OxMg/L and OxML/L due to incorrect

initialization of buffers.

- Incorrect column names for TimeS, TimeH, SndVelC, SndVelD, SndVelW, DepthF and

DepthS.

2. MODIFICATIONS:

SEACON:

------

- Allow up to 36 words for SBE31.

SEAPLOT:

-------

- Make <Enter> the exit-key for Min-Max values subform.

3. NEW FEATURES:

SEACON:

------

- Add "none" to pressure type selections.

- Add 6 scans/sec to scan-rate selections.

SEASAVE:

-------

- Add command line options -v1 and -vl for computing a set of variables for

SBE31.  The set contains delta-temp/delta-time, standard-deviation-of-primary-temp, and

secondary-temp - primary-temp.

NOTES ON SEASOFT VERSION 4.012

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- Negative exponent values and fluorometer values occupy 12 columns in .CNV files

causing two columns combined together.

DATCNV, DERIVE, ROSSUM, SEASAVE:

-------------------------------

- Incorrect oxygen ml/l and mg/l values if conversion unit is ENGLISH.

DERIVE:

------

- It doesn't work for centered data (doc/dt or dz/dt).

SEASAVE:

-------

- Incorrect variable names for density, sound velocity, depth and time

in the bottom status line.

TERM1621:

--------

- The program blows up due to limited internal buffer length.

NOTES ON SEASOFT VERSION 4.013

==============================

1. BUG FIXES:

SEAPLOT:

-------

- <Esc> doesn't terminate plotting job on HP plotter gracefully.

- Incorrect min max labels for y variable on plots other than the first page.

- When plotting multi-pages to HP printers or plotters, it only plots the last

page and does screen-dump for other pages.

2. MODIFICATIONS:

SEACON:

------

- Increase decimal digits from 2 to 3 for temperature coefficient f0.

NOTES ON SEASOFT VERSION 4.014

==============================

1. BUG FIXES:

2. MODIFICATIONS:

SEACON, SEASAVE:

---------------

- Only check the last character in conductivity sensor serial number

to determine if the conductivity sensor is wider range or not.

The conductivity sensor is wider range sensor if the last character is

'w' or 'W'.  This last character is dropped when do the matching.

- Increase decimal digits for conductivity values to 6.

- Rename variable NEPHLOMETER to NEPH_IFREMER.

3. NEW FEATURES:

BINAVG:

------

- Add an new field target-value into surface bin form.

SEACON, SEASAVE:

---------------

- Add two new sensors NEPH_CHELSEA and ZAPS for SBE911.

NOTES ON SEASOFT VERSION 4.015

==============================

1. BUG FIXES:

DATCNV:

------

- Command line option -ax doesn't work correctly. It only processes one .hex file.

SEACON, SEASAVE:

---------------

- The programs hangs when editing DOUBLE fields on computers such as EPSON equity

I+ and HP vectra.

2. MODIFICATIONS:

DATCNV, SPLIT, STRIP:

--------------------

- Make <Esc> to quit the whole process when in batch mode.

NOTES ON SEASOFT VERSION 4.016

==============================

1. BUG FIXES:

BINAVG:

------

- Zero surface bin target is treated as non-zero.

- Incorrect max-pressure-scan-number when skip scans.

DATCNV:

- Incorrect depth values for non-zero latitude.

DATCNV, SEASAVE:

---------------

- Incorrect initial values for OxMg/L, OxML/L, dt/dt, and dz/dt.

SEACON:

------

- Pressure voltage is missing in the summary display for SBE25.

SEAPLOT:

-------

- Command line option -i doesn't work.

- It doesn't recognize the bad flag in binary files.

SEASAVE:

-------

- If the user turns off the power on the deck-unit in the middle of the

realtime process, the computer hangs.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Increase decimal digits for BOC and SOC from 3 to 4.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Upgrade to MSC C/C++ V7.00 compiler and RTLink linker. Start using

overlays for SEASAVE and using .RTLs for other programs.

- Add an new variable OXSAT (oxygen saturation).

DERIVE:

------

- Add an new command line option -v1 to display secT-priT (secondary temp-

primary temp) and secC-priC (secondary conductivity - primary conductivity).

SEASAVE:

-------

- Add an new variable secC-priC (secondary conductivity - primary conductivity).

NOTES ON SEASOFT VERSION 4.017

==============================

1. BUG FIXES:

All TERMINAL Programs:

---------------------

- When the user chooses not to output the user-defined-header, the line

"System UpLoad Time" will not be included in output .hex files.

DATCNV, SEASAVE(archived):

-------------------------

- Cannot get correct start-time for some data files.

DERIVE:

------

- When the input .CNV file contains only temperature without conductivity,

the user cannot select a correct type of density and the density values

in the output .CNV file will be all zeros.

SEACON:

------

- Incorrect total number of bytes for SBE25.

SEAPLOT, SEASAVE:

----------------

- The background color changes to gray when dumping screen without grid lines.

SEASAVE:

-------

- Incorrect pressure values for SBE17 with strain gauge when number-of-scans-

averaged < 8.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Change exponent format from e+|-xxx to e+|-xx.

- Modify form editing to make <Enter> to goto next field and <Ins> to toggle

the insert mode and overtype mode in STRING editing.

3. NEW FEATURES:

SEAPLOT, SEASAVE:

----------------

- Add an option to have dotted or connect grid lines.

NOTES ON SEASOFT VERSION 4.018

==============================

1. BUG FIXES:

DATCNV:

------

- Incorrect OxML/L values when variable OxMg/L was not selected.

- Incorrect initial oxygen values for SBE19 profilers.

- Incorrect last line of data for SBE19 in output .CNV files if the DATCNV

process is interrupted.

SEAPLOT:

-------

- If plot a group of short .CNV files, the plots will be terminated in the

middle.

SEASAVE:

-------

- Incorrect pressure and voltage values for SBE17 if the firmware version >= 2.0.

2. NEW FEATURES:

DERIVE:

------

- Add an new variable DM (dynamic meters).

SEAPLOT:

-------

- Add an new command option -ni to not initialize the HP plotter.

NOTES ON SEASOFT VERSION 4.019

==============================

1. BUG FIXES:

ASCIIOUT:

--------

- Command line option -o doesn't work.

DATCNV:

------

- Cannot skip over scans for SBE19.

- Incorrect pressure-temperature values for SBE19 profilers when the firmware

version < 3.0.

SEASAVE:

-------

- Incorrect dz/dt, dox/dt, and dt/dt values when skip scans between computations.

- Disk access and disk full error messages are not displayed when in XY graphical

display mode.

2. MODIFICATIONS:

ALIGNCTD:

--------

- Modify the program to make the input and output .CNV files contain same number

of data lines.

3. NEW FEATURES:

ASCIIIN:

-------

- Initial release.  It adds header information into .ASC files and output them

as .CNV files.

DATCNV:

------

- Add scan-number column to .ROS files if it is not already in the set of selected

variables.

DATCNV, SEASAVE:

---------------

- Add an new command line option -v2 for an new variable "plume anomaly".

- Add julian-days to the time selections.

SEAPLOT:

-------

- Add command line options -v for overlay and -w for waterfall.

- Add interactive overlay feature.  <Ctrl_F7> is designated to initialize

overlay display.

NOTES ON SEASOFT VERSION 4.020

==============================

1. BUG FIXES:

ASCIIIN, DERIVE, STRIP:

----------------------

- Variable indices are incorrect for files other than the first one when in

batch mode and the setup form is displayed.

BINAVG:

------

- Command line option -vd<binSize>, used to indicate using depth bin and to

specify the bin size, doesn't work.

ROSSUM:

------

- The output values are incorrect except the ones in the first bin.

SEACALC:

- Incorrect gravity values.

SEAPLOT:

-------

- PAR values are displayed in linear scales rather than in log scales.

SEAPLOT, SEASAVE:

----------------

- During archived XY-display, press <F1> key will freeze the screen.

SEASAVE:

-------

- Incorrect start-time for SBE17 during archived display.

- The <F1> key doesn't work after realtime display.

- During realtime display, marked data are sent to the .mrk file once only.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Make input path and output path include the full path when <Enter> is pressed.

- Upgrade RTLinkPlus to V5.20.

BINAVG:

------

- Allow BINAVG to process .CNV files containing non-time interval.

SEASAVE:

-------

- Modify serial-output so it can send either Pressure or Altimeter-Height values

to SBE14 remote display.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add an new choice "density" to variable DENSITY.

ASCIIIN:

-------

- Add an new field "Scan Interval Variable" into the form.  This field is used to

select the interval variable which can be time, pressure, or depth.

BUOYANCY:

--------

- Initial release.  It computes buoyancy frequency and stability frequency values.

SEASAVE:

-------

- Add number-of-voltage-words into SBE911's output data files' header.

WFILTER:

-------

- Initial release.  It is window filters.

NOTES ON SEASOFT VERSION 4.021

==============================

1. BUG FIXES:

SEASAVE:

-------

-In fixed display, when total number of variables are less than 9 and

variables are separated by "none", the variables following "none" will

not show up in the display.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Rename the temporary file from TEMP to TEMP.CNV to avoid confusion with

possible subdirectory named TEMP.

SEAPLOT, SEASAVE:

----------------

- Change the precision for graphical display min and max values to be based

on the variable types.

SEASAVE:

-------

- Modify to not suppress the added surface PAR word.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add new variables LATITUDE, LONGITUDE, AND NEWPOS.

ASCIIOUT:

--------

- Add an command line option -lp and an option to the form to separate

input files to pages and label each page.

BINAVG:

------

- Add an new command line option -c<number> for selecting the first bin's

center value for pressure bin or depth bin.

SEACON:

------

- Add an new flag field to the SBE911Plus setup form to indicate that surface

PAR voltage word is added to the input scans by SBE911Plus.

NOTES ON SEASOFT VERSION 4.022

==============================

1. BUG FIXES:

CELLTM:

------

- When it processes an input .CNV file which was processed by ALIGNCTD before,

CELLTM gets "float overflow" error on some computers.

- If a .CNV file contains only secondary temperature and secondary conductivity,

the conductivity values will not be changed.

- If a .CNV file contains only primary temperature and primary conductivity,

using -2 option will cause run-time floating point value error on some

computers.

SEAPLOT:

-------

- When display contour data for invalid ranges, some contour lines will show

up unexpectedly.

SEASAVE:

-------

- It sometimes hangs when display archived data for SBE17.

2. NEW FEATURES:

CELLTM:

------

- Add an new command line option -s for using the secondary temperature to

compute the secondary conductivity.

NOTES ON SEASOFT VERSION 4.023

==============================

1. BUG FIXES:

All TERMINAL Programs:

---------------------

- The second line in the header of a data file will be concatenated to the

first line if user-defined header is entered.

2. MODIFICATIONS:

ASCIIOUT:

--------

- Delete the command line option -f and remove the field in the main form

for output the flag column.

DATCNV, SEASAVE:

---------------

- Modify them so they can handle the data files with the first line and the

second line in the header combined.

SEACON:

------

- Modify it for SBE31 to give the selections for primary and secondary

temperatures and conductivities even if there are less or equal to 3

frequencies.

3. NEW FEATURES:

ASCIIIN:

-------

- Add new fields to the form for user-defined variables' descriptions and

number of decimal spaces.

ASCIIOUT:

--------

- Add a subform for selecting output variables.

- Add a subform for adding an user defined first column.

SEAPLOT:

-------

- Add choices "seconds" and "hours" to the list of input variables in

addition to scan-number if time[seconds] is not among the input variables.

SEASAVE:

-------

- Add an new variable NBYTES for number of data bytes processed so far.

NOTES ON SEASOFT VERSION 4.024

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- The bad flag is stored as 0.000 rather than -9.99e-29.

2. MODIFICATIONS:

ROSSUM:

------

- New ROSSUM with forms.

NOTES ON SEASOFT VERSION 4.025

==============================

1. BUG FIXES:

All TERMINAL Programs:

---------------------

- Sometimes get upload files from wrong directories.

WFILTER:

-------

- It will not be able to create the wfilter-type subform after selecting

several input files.

2. MODIFICATIONS:

SEASAVE:

-------

- Prompt for data-start-time for .DAT files only when displaying julian

time.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Display the percentage of process completed and the elapsed time.

- Add an new choice "micromoles/kg" to OXYGEN selections.

- Add secondary oxygen, add userpoly0, userpoly1, and userpoly2.

- Make calibration date an additional coefficient to all sensors.

ASCIIOUT:

--------

- Add fields for first-column-name and column-separator.

- Add command line options -1, -1x, -1n<firstColumnName>,

-1v<firstColumnValue>, and -s<separator>.

DATCNV:

------

- Check the temperature and conductivity sensor serial numbers in data files

to make sure they match the ones in .CON files. This has been done so far

for SEASAVE archived data only.

- Add an option to output both upcast and downcast or downcast only.

DERIVE:

------

- Add .CON file list to the main form and add an new command line option

-c<filename> to specify the .CON file name.  The .CON file is used to

get oxygen coefficients.

SEAPLOT:

-------

- Add LPT1 and LPT2 to the plotter port choices.

SEASAVE:

-------

- Display Rosette-Bottle-Firing Sequence on the side for both ros-1015 and

ros-1016 during realtime XY-display.

TERM17, TERM25:

--------------

- Add fields into set-up form for temperature-sensor serial number and

conductivity-sensor serial number.  These sensor serial numbers were

getting from seasoft.con before.

NOTES ON SEASOFT VERSION 4.026

==============================

1. BUG FIXES:

DATCNV:

------

- Incorrect hex data scan length for SBE19.

2. MODIFICATIONS:

DATCNV:

------

- In batch mode default input file extension to .DAT for SBE911, SBE911Plus,

SBE911e, and SBE31, and default to .HEX for other instruments.

NOTES ON SEASOFT VERSION 4.027

==============================

1. BUG FIXES:

DERIVE:

------

- It hangs when there are .CON files but not .CNV files in the input path.

DATCNV:

------

- It doesn't work correctly  for downcast only.

- The output .CNV file contains extra scans for SBE19 profilers if the

scan interval is greater than 0.5 seconds.

2. MODIFICATIONS:

SEAPLOT:

-------

- Default the plotter COMM port to LPT1.

- Modify plotter-set-up form and move plotter-selection field to plotter-set-up

subform for clarity.

3. NEW FEATURES:

ASCIIOUT:

--------

- Add command line options -f and -fx to output|not-output the flag column;

add -ca to output all the data columns excluding the flag column.

DATCNV, SEASAVE:

---------------

- Add latitude, longitude, and new-position into the variable list for

SBE911Plus if NMEA interface is installed.

- Add command line option -v4 for SBE16 with digiquartz with temperature-

compensation to have 4 bytes for the pressure values; add command line

option -v5.

DATCNV, SEACON, SEASAVE:

-----------------------

- Add an new command line option -v3 to flag IOW.

SEACON:

------

- Add an new field to SBE911Plus form as "SBE911Plus NMEA interface is installed =".

SEAPLOT, SEASAVE:

----------------

- Enable <Ctrl_F9> to dump screen or plot the display during pause.

SEASAVE:

-------

- Add an new command line option -a<autoFireFileName> to specify the auto-fire

file name.

- In realtime mode, if the NMEA interface is installed, add a field to the

misc-run subform as "Store Lat/Lon in Raw Data File = " and add this line

to the header in output data files.

NOTES ON SEASOFT VERSION 4.028

==============================

1. BUG FIXES:

FILTER:

------

- Incorrect number of lines in output .CNV files.

SEASAVE:

-------

- After displaying data from a file without latitude/longitude information,

the first time display data from a file with these information will get

wrong latitude and longitude values.

2. MODIFICATIONS:

DATCNV, SEASAVE:

---------------

- Delete command line option -v5 for SBE16.

3. NEW FEATURES:

SECTION:

-------

- Add a field to the set-up form to allow the user to select number of

scans to skip form the end in the input file.

NOTES ON SEASOFT VERSION 4.029

==============================

1. MODIFICATIONS:

DATCNV, SEASAVE:

---------------

- For SBE16 with pressure sensor type as digiquartz with temperature-

compensation and with -v4 selected, the frequency #2 and pressure

values are computed with higher precision.

NOTES ON SEASOFT VERSION 4.030

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- User-defined variables' values were displayed without decimal point.

- The trailing blanks in sensor serial numbers caused matching problems.

ASCIIOUT:

--------

- Incorrectly output flag-column while in non-batch mode.

- The conversion between time-string and seconds during daylight-saving

time were incorrect.

DATCNV:

------

- For SBE19 profiler when output downcast only, the scan numbers and the

split count were incorrect.

- For SBE19 profiler when the conductivity sensor type was other

than STANDARD, the conductivity values from scans before the first

reference scan were incorrect.

ROSSUM, SEASAVE:

---------------

- The conversion between time-string and seconds during daylight-saving

time were incorrect.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Separate the input file name and output file name to two lines

in the message when starting a program.

- Upgrade linker to RTLinkPlus VERSION 6.10.

SEACON:

------

- Delete fields for ad590m and ad590b from the calibration-coefficient

form for digiquartz without T/C.

- Modify the prompt for Fluorometer-Chelsea's offset, change the

nominal value from 0.0 to 2.0.

SEAPLOT:

-------

- Enable <Ctrl_F7> during pause of display. This allows you to overlay partial

screen displays.  Also modify the main message accordingly.

SEAPLOT, SEASAVE:

----------------

- Change the key stroke and messages for GRAFPLUS screen dump back to

press <Ctrl-F9>. Change status line color.

- Change Y label's direction to upwards.

TERM1621, TERM19, TERM25:

------------------------

- Change default baud rate for SBE16, SBE19, and SBE25 to 600.

3. NEW FEATURES:

All SEASOFT Programs:

- Allow SBE16 to have up to two external frequencies, and the second external

frequency is defaulted to secondary conductivity.

All TERMINAL Programs:

---------------------

- Add 19200 and 38400 into the group of communication baud rates to try when

establishing the connections between the computers and the SBE instruments.

ASCIIOUT:

--------

- Add an new field to the set-up form to offer choices of converting

julian days to a format as HH:MM:SS, HH:MM:SS.SS, MMM DD YYYY HH:MM:SS,

DD MMM YYYY HH:MM:SS, or don't convert.

DATCNV:

------

- Add option -h to replace the header part in .dat file by .hdr file's

contents.  This allows you to modify the header part of binary files.

- Add new fields to the form for all the instrument types.  These fields

are water-bottle-sampling set up, file creation options (output .CNV

file only, .ROS file only, or both), and source of water-bottle scan

ranges.  It allows you to choose the offset and duration for scan ranges

if water-bottle scan ranges are from the input data file.

DATCNV, SEACON, SEASAVE:

-----------------------

- Add variables latitude, longitude, and new position for SBE16, SBE19,

and SBE25.

MARKSCAN:

--------

- Initial release. This program reads data from .mrk files and output water

bottle scan ranges for each bottle into .bsr files.

ROSSUM:

------

- Add an new field to the set-up form and add an new option -m[x] to

include|don't include MIN and MAX values for variables to be

averaged. Also label the values at the end of each line.

SEAPLOT:

-------

- Add an color-selection option to both overlay and waterfall. The syntax is

as follows:

The option -c<varIndex><colorIndex> can be added after a file name in an

overlay file, or after an optional label in a waterfall file.  The varIndex

is the variable index within range [1,4] and the colorIndex is the color

index within range [0,14].  Up to 4 color options can be specified for

variables' color.  The colorIndex mapping is:

0: Blue;          1: Brown;          2: Cyan;        3: Dark-Gray;

4: Green;         5: Intense_White;  6: Light_Blue;  7: Light_Cyan;

8: Light_Green;   9: Light_Magenta; 10: Light_Red;  11: Magenta;

12: Red;          13: White;         14: Yellow.

SEASAVE:

-------

- Add new fields to both REALTIME and ARCHIVED forms to allow you to save

the display parameters to .DSP files while editing the forms.

- Add LPT1 and LPT2 to the remote display COMM port selections.

- Add option -m[x] to show|don't show red mark lines for water bottle firings

during archived display.

NOTES ON SEASOFT VERSION 4.031

==============================

1. MODIFICATIONS:

ROSSUM:

------

- Increase local array sizes from 120 to 1440 to allow it contain up to 1 minute

of data at 24hz.

NOTES ON SEASOFT VERSION 4.032

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- Fix bug in Fluorometer, turner computation.  In SEACON, separate turner sensor to

two types - turner fluorescence sensor and turner range sensor.  In other programs

compute the turner values based on both these sensor values.

ASCIIOUT:

--------

- Cannot change and save the output choice for the flag column in the main setup form.

DERIVE:

------

- Incorrect computation for oxygen micromoles /kg.

SEASAVE:

-------

- Fix bug with processing external volts for SBE21 when using secondary temprature

sensors.

WFILTER:

-------

- Fix bug in filter-type subforms.  Sometimes after selecting several different input

.CNV files, the filter types' subforms get messed up.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Add coefficient t4 into digiquartz pressure computation.

DERIVE:

------

- Change output format for oxygen values from 11.3f to 11.5f.

SEASAVE (realtime):

------------------

- When the incoming scans have shorter length than expected, SEASAVE will show display

error messages and quit data acquisition.  This error could be caused by using a wrong

.CON file or the configurations in the .CON file are incorrect.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add command line option -e<cfgFileName> for configuration file name to replace the

default <progName>.CFG.

- Add command line option -t<tempFileName> for temporary file name to replace the

default TEMP.CNV.

DERIVE, SEASAVE:

---------------

- Add new variables DIFFD for secondary density - primary density (sigma-theta) and DIFFS

for secondary salinity - primary salinity when command line option -v1 or -vl is specified.

SEASAVE:

-------

- Add DEPTH into remote display variable type choices.

- Add SBE carousal to water sample choices in realtime mode. The firing choices are user

input and sequential (no offset choice).

NOTES ON SEASOFT VERSION 4.033

==============================

1. BUG FIXES:

DATCNV:

------

- When output 7 or 8 variables and create both .CNV and .ROS files, sometimes

there is an extra tab before the line "# nvalues =" in the .CNV file, sometimes

the line "# nvalues =" is appended to the previous line in the .ROS file.

- Lat/lon data in reference scans for instrument type SBE19 are not converted

correctly.

SEASAVE (realtime):

----------------

- It doesn't work for all instrument types with NMEA interface installed when the

NMEA message mode is either GGA or GLL.

2. MODIFICATIONS:

SEASAVE (realtime):

------------------

- Speed up communication initialization for SBE11Plus with RS232 connection.

- Change the status line variable display to be left-justified.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add dOC/dT into the choices for variable oxygen.

NOTES ON SEASOFT VERSION 4.034

==============================

1. BUG FIXES:

CFGTOCON:

--------

- The configurations in output .CON files are incorrect when the pressure type is STRAIN

GAUGE and instrument type is other than SBE19 Profiler.

DATCNV:

------

- It cannot convert SBE19 Profiler's .HEX data files which contain only one set of reference

scans with only one data scan before the reference scans.

- It cannot run with -a or -ax option when output .ROS files.

SEAPLOT:

-------

- After screen dump the bottom line disappears.

SEASAVE (realtime):

------------------

- For SBE11plus with RS232 connection, sometimes the Go-Log command is not executed

causing the deckunit's buffer overflow (this only occurs in V4.033)

- When change instument type from non-sbe911 types to sbe911 types (SBE911, SBE911e and

SBE911plus) by selecting diffent .CON files, the number-of-scans-averaged field in

Misc-Run-Parameters subform doesn't appear.

- The parameter number-of-seconds-between-screen-updates is not used in the realtime display.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Update the system time on the top line when it is not in graphical display.

DERIVE:

------

- Variables Acceleration and dZ/dT will be included in the variable list only when Pressure

is among the input variables.

3. NEW FEATURES:

SEAPLOT:

-------

- Add an new field into the main form to let you be able to save the plot parameters to .PLT

files while editing the form. This feature is similar to the one in SEASAVE.

NOTES ON SEASOFT VERSION 4.035

==============================

1. BUG FIXES:

ASCIIIN:

-------

- It blows up occasionally especially after selecting several different files.

ROSSUM:

------

- It sometimes gets out-of-memory error.

2. MODIFICATIONS:

ASCIIIN, ASCIIOUT, FILTER, SEAPLOT, STRIP, WFILTER, WILDEDIT:

------------------------------------------------------------

- Modify them so when the input-path changes the input-variable subform will update

accordingly.

3. NEW FEATURES:

DATCNV, DERIVE, ROSSUM:

----------------------

- Add command line option -s to indicate use same file names for both input data files and

.con files in batch mode.

SEASAVE:

-------

- Add command line option -b<markLineType><markLineColor> for bottle firing mark

line's type and color. The mark line's type can be 0 for connected line and 1 for dotted

line; the mark line' color mapping is as follows:

0: Blue;          1: Brown;          2: Cyan;        3: Dark-Gray;

4: Green;         5: Intense_White;  6: Light_Blue;  7: Light_Cyan;

8: Light_Green;   9: Light_Magenta; 10: Light_Red;  11: Magenta;

12: Red;          13: White;         14: Yellow.

For example -b17 means the mark line is dotted line and its color is light cyan.

SEASAVE (realtime):

------------------

- Mix the automatic and manual firing modes, i.e., the user can press <Ctrl_F3> or press

<Fire> button on the deck unit to fire bottles during automatic firing.

- Designate hot key <Shift_F3> to enable automatic firing's upcast.

- Set data bits for firing bottles when the <Fire> button is pressed, so pressing <Ctrl_F3>

and pressing <Fire> button on the deck unit to fire bottles will function the same.

NOTES ON SEASOFT VERSION 4.200

==============================

1. BUG FIXES:

DERIVE, ROSSUM:

--------------

- After selecting .con files, the sub-choices for Density, Salinity etc. in the Variables-

to-be-Derived Form are not corresponding to the input variable selections in the input

file anymore.

ROSSUM:

------

- The sub-choices for Density, Salinity etc. in the Variables-to-be-Derived Form weren't

highlighted correctly.

2. MODIFICATIONS:

SEASAVE:

-------

- Change "Number of Pts to Compute between Status Line Updates" to "Number of Seconds

between Status Line Updates".

- Show bottle numbers rather than sequence numbers on bottle firing mark lines.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add an new variable modulo error count.

- Add an new variable ITS-90 and rename the original temperature to IPTS-68.

- Add an new sensor Biospherical natural fluorometer.  Add new variables

natural fluorescence, production, and chlorophyll concentration.

DATCNV:

------

- Add an new choice "Bottle Log File [.BL]" to the Source of Scan Range menu

for water bottle firing.  The .BL files can be used to specify the water

bottle firing ranges for instruments such as SBE19 and SBE25 since .hex data

files don't contain any water bottle firing informations.

SEASAVE:

-------

- Add LPT1, LPT2, and LPT3 to ascii output choices.

- Add "Number of Seconds between ASCII Data Updates" and "Number of Seconds between

Remote Display Updates" to ASCII ouptut form.

- Add mark type, mark line type, and mark line color to real-time form's misc-run-

parameters subform.  These mark parameters are used for both real-time marks and

bottle firing marks.

NOTES ON SEASOFT VERSION 4.201

==============================

1. BUG FIXES:

SEASAVE:

-------

- When command line option -v1 is selected, sometimes got "sqrt DOMAIN" error.

- In archived graphical display, sometimes the fired bottle numbers displayed on

the screen are all zeroes.

NOTES ON SEASOFT VERSION 4.202

==============================

1. BUG FIXES:

ROSSUM:

------

- Cannot derive oxygen's docdt and mm/kg.

2. MODIFICATIONS:

ALIGNCTD:

--------

- Take away command line option -2.

DATCNV, DERIVE, ROSSUM, SEASAVE:

-------------------------------

- Change oxygen's computation to mg/l = ml/l * 1.4291.

SEAPLOT:

-------

- Allow the user to enter blank lines in overlay or waterfall files.

SEASAVE:

-------

- Reduce seasave.exe's ram size by 30k.

SEASAVE (real-time):

-------------------

- Report all the bottle firings and RESET commands to .BL files.

- Display all the bottle firings on X-Y plot.

- For instrument type SBE16, 19, 21 and 25 with Carousel, modify COMM port subform

to let user enter the baud rate between SBE33 and the computer and the baud rate

between SBE32 and CTD.

- Change the output option's default from "Overwrite the File" to "Output to a

Different File" to avoid the input file to be overwritten accidentally.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add LPT3 to line printer's selections.

- Add following units choices for conductivity: Siemens/meter, milliSiemens/centimeter,

and microSiemens/centimeter.

- All the user to use either primary or secondary temperature to compute potential-

temperature.

ALIGNCTD:

--------

- Add new fields into the main setup form to allow the user to align primary and

secondary temperature, conductivity, and oxygen independently.

DATCNV:

------

- Store all the configured sensors' serial numbers in the header part of the output

.cnv files.

SEASAVE:

-------

- Add a field in "ASCII Output Set Up" form to prompt for Minimum Pressure to Enable

Altimeter Alarm.  Set the altimeter's alarm only when the pressure is greater or

equal to the minimum pressure.

TRANS:

-----

- Add command line option -a and -ax for batch process.

NOTES ON SEASOFT VERSION 4.202a

==============================

1. BUG FIXES:

SEASAVE (realtime):

------------------

- Enable <Ctrl_F2> and <Ctrl_F4> keys only when the instrument type is either SBE911plus

or SBE911e with any types of water bottles installed.

NOTES ON SEASOFT VERSION 4.203

==============================

1. BUG FIXES:

SEASAVE:

-------

- Cannot dump screen while output ascii data to LPTx.

2. MODIFICATIONS:

DATCNV:

------

- If the input .DAT file has a companion .NAV file, add the .NAV file data to the

output .CNV file's header.

DATCNV, DERIVE, ROSSUM:

----------------------

- Modify the display messages to align input file names and output file names.

LOOPEDIT:

--------

- Add options for specifying minimum velocity.  The options are:

1. Fixed minimum velocity or

2. Percent of mean speed.  If this option is selected, prompt for the window size

for mean speed's calculation and the percentage of the mean speed to be used in

the calculation of the minimum velocity.

SEASAVE (realtime):

------------------

- Add more options for storing Lat/Lon information in files. The options are:

1. Add to header only.

2. Add to header and append to every scan or

3. Add to header and append to .NAV file when <Ctrl-N> is pressed.  When <Ctrl-N>

is pressed and this option is selected, write a line to the specified .NAV file

in the format of time, lat, lon, pressure, and scan number. The time here is

the NMEA time if available, otherwise, is the system time.

- Add <Ctrl-P> to Pause|Unpause the Ascii Output if the output device is LPTx.

TRANS:

-----

- Indicate the input .CNV file's type (binary or ascii) on the main setup form.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add an new sensor Sea Tech LS6000.

DATCNV, DERIVE, SEASAVE:

-----------------------

- Add an new variable Average Sound Velocity.

NOTES ON SEASOFT VERSION 4.204

==============================

1. BUG FIXES:

DATCNV:

------

- SBE19: the first two converted scans contain incorrect temperature and

conductivity values if the first two scans in the input data file are reference

scans.

ROSSUM:

------

- Incorrect time strings in the output .btl files.  This was caused by a

local value declared as an interger rather than a long.  Also ROSSUM

did not skip over "RESET ...." lines in the .btl file.

SEASAVE (realtime):

------------------

- SBE 31: occasionally hung when starting to acquire data.  SEASAVE was sending

XOFF at the wrong time.

2. MODIFICATIONS:

ALIGNCTD:

--------

- Regroup the form fields in the setup form.

DATCNV:

------

- Display error messages for SBE19 when DATCNV cannot find reference scan pairs

in input data files.

DATCNV, SEASAVE:

---------------

- Allow pressure frequency for "paros without TC" (in addition to "paros with TC") to

have four bytes for instrument SBE16 when a command line option -v4 is specified.

LOOPEDIT:

--------

- Add "Total Number of Bad Scans So Far = " to the display messages.

TERMINAL Programs:

-----------------

- Change drives to directory paths for all the input and output files.

3. NEW FEATURES:

SEASOFT All Programs:

--------------------

- SBE31: Add support for NMEA interface.

SEACON:

------

- Add sensor types IFREMER and NEPHELEMETER to SBE19 and SBE25.

SEASAVE:

-------

- Add "table driven" to the bottle firing choices.

NOTES ON SEASOFT VERSION 4.205

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- Variables dz/dt and accel have incorrect values when using English units.

DATCNV:

------

- It freezes out occasionally when converting data for SBE19 with strain gauge

pressure sensor.

- it doesn't output sensor serial numbers to the output .cnv file's header correctly

when processing the same .hex file more than once.

SEASAVE (realtime):

------------------

Older version of SBE 31 sending four bytes per word instead three to the computer.

Terminal Programs:

-----------------

- Default upload files don't have file extensions.

- Programs freeze out due to upload path wasn't read from the .cfg files correctly.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Add and modify all variables' units to be SI units.

- Modify turner fluorometer sensors' descriptions.

- Change chelsea fluorometer's computation.

SEASAVE (realtime):

------------------

- Improve GPIB error messages.

3. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add an new type of turner fluorometer: turner 10-Au.

ROSSUM:

------

- Add command line option -b<bottleSNFileName>.  If bottle.sn file is in the current

directory or bottleSNFileName is specified, output bottle S/Ns to the .btl files.

SEACALC:

-------

- Add additional choices to the main menu for displaying seawater units and for

autosal calculation.

SEASAVE:

-------

- Add baud rate 19200 and 38400 to ASCII output's serial port's baud rate choices.

TERMAFM:

-------

- This is an new program.

NOTES ON SEASOFT VERSION 4.206

==============================

1. BUG FIXES:

TERMAFM:

-------

- The Min-Pressure and Pressure-Window-Size etc. are not set correctly when program

the AFM if the AFM is in Stationary Mode.

TERM25:

------

- It doesn't work when uploading all data as one file or by scan number range

when the upload baud rate differs from the communication baud rate.

2. MODIFICATIONS:

SEASAVE:

-------

- Change one of the choices for the Printer Control Selection from "Others (GRAFPLUS

Control)" to "Others (DOS GRAPHICS or other memory resident printer drivers)".

SEAPLOT:

-------

- Change it to stand alone overlay structure to reduce the memory requirement about

40k.

3. NEW FEATURES:

SEACON, SEASAVE:

---------------

- Add an new type of sensor - Bottom Contact Switch for instrument types SBE19 and SBE25.

NOTES ON SEASOFT VERSION 4.207

==============================

1. BUG FIXES:

DATCNV:

------

- If a .BL file is selected to get scan range information for bottle firing,

and the .BL file contains "RESET" lines, the Number of Bottle Fired in both

output .CNV and .ROS files are incorrect.

SEAPLOT:

-------

- When the process of sending data to a plotter gets interrupted, sometimes

extra lines will show up on the screen and the top message will be wrong too.

- SEAPLOT may get confused when plotting several one-page plots in a batch mode

and "Print When Reach Bottom of Plot or End of Data Set" is selected.

- When plotting more than 7 plots in a batch job on some plotters, a data

overflow on the plotter might occur.  Use the new option -p<pauseTime>

(see details in the following NEW FEATURES section).

SEASAVE:

-------

- When -i<inFileName> is used, the input file name specified here is not showing

at the bottom right corner on the screen, instead, the one in the seasave.cfg

file is displayed.

TERMAFM:

-------

- The pressure offset was computed as psia rather than decibars

2. MODIFICATIONS:

All TERM Programs:

-----------------

- Terminal programs no longer automatically wake the instrument when the program is run.

Press <F6> to wake the instrument up and establish communications.

- When <F6> is pressed, the program uses only the chosen COMM port to establish

the serial connections, it does not try other COMM ports.

All SEASOFT Programs:

--------------------

- Programs will allow the user to go to the setup-form even if the selected input file or .con

file are invalid.

- Enable the user to quit a batch process if <Esc> is pressed during the batch

process. All the user has to do to use this feature is to modify the batch file

by adding a line "if errorlevel 1 goto quit" after running each program and

adding a line ":quit" to the end of the batch file.

SEASAVE:

-------

- Add the data file name (input file name for ARCHIVED display and output file

name for REALTIME display) and time to the ASCII optput.

SEASAVE, SEAPLOT:

----------------

- Modify the disk access error or printer error messages during X-Y graphical display.

When an error occures then one presses either <Enter> or <Esc> key, the error message

will be erased before the process continues.

TRANS:

-----

- When select a different input file path, the input-file-type field is also updated.

WFILTER:

-------

- When select a different input file path, the scan-interval field is also updated.

3. NEW FEATURES:

All SEASOFT, TERM Programs:

--------------------------

- Add COM3 and COM4 to the serial port choices.

SEAPLOT:

-------

- Map screen display's colors to plot's colors on HP color printer as close

as possible.  The color mapping will work on all types of HP printers as

long as the type of HP printer supports HP-GL/2.

- Add an new command line option -p<pauseTime>. This indicates to pause pauseTime

seconds after sending data to a plotter in a batch mode. If you use -p without

specifying the pauseTime, the default 60 seconds pause will be used. This option

is useful especially when you have to plot a lot of plots in a batch job.

NOTES ON SEASOFT VERSION 4.208

==============================

1. BUG FIXES:

DATCNV, SEASAVE:

---------------

- For SBE17 or SBE17plus if the input file's path in the .hex file's header

constains the word "SEARAM", the output pressure and pressure-temperature

values are computed incorrectly. This bug originated in version 4.020.

- When the sensor serial numbers in .con file don't match the ones in the input

data file and the user selects another .con file to continue, sometimes the

program hangs.

SEASAVE:

-------

- In X-Y plots a large positive number was sometimes displayed as a large

negative number.

TERMAFM:

-------

- When pressing <F8> to switch from SBE19/25 to AFM or vice versa, the instrument

wouldn't wake up correctly.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Improve the error handling for when there are no input files in

the specified directory.

SEACON:

------

- SBE 25: Add a field to specify real time data output rate.

SEAPLOT:

-------

- Eliminate subforms for variables' min and max values. A variable's min and

max values will be shown above the variable's min or max value field when

the min or max value field is being edited. This change enables SEAPLOT's

forms to have the similar look and feel as SEASAVE's.

SEASAVE:

-------

- Remove the command line option -s since it is no longer useful.

SEASAVE (realtime):

------------------

- Enable the user to go back to the main menu after editing the user-header form.

- Modify the Auto-Bottle-Fire logic. Enable the user to choose firing which bottle

at which pressure by editing the auto-fire configuration file. The pressure and

bottle position lines have the following format:

pressure, bottle position, 1 | 2

For 1015 only the pressure field is required and used, and the bottles will

always be fired sequentially. For 1016 and SBE-Carousel, one or two fields can

be specified. If only the pressure field is specified, bottles will be fired

sequentially, otherwise, if the bottle-position field is also specified, the

specified bottle will be fired at the chosen pressure. The third field 1 | 2

is not used by 1016 and SBE-Carousel thus can be omitted. For Hydro-Bios and IOW,

the third field 1 | 2 indicating single or double fire, the bottle-position field

is not used but needs to be there as a place holder.

3. NEW FEATURES:

ALIGNCTD:

--------

- Add an new command line option -v<colNumber>,<advSeconds> for more advanced users.

The option lets the user specify the column number and the number of seconds to

advance.  For example, -v0,-3.5 means to advance the variable values at column #0

by -3.5 seconds.  Note that no spaces are allowed between these parameters. These

values override the ones originally specified in alignctd.cfg. This option can be

used more than once.

DATCNV:

------

- For each frequency, voltage, pressure channel add a line of descriptions to

the output .cnv file's header.  The line contains channel name, sensor type,

sensor serial number and calibration date.

DATCNV, SEACON, SEASAVE:

-----------------------

- Add choices to temperature computation: using a, b, c, d, f0 or using g, h,

i, j, f0 coefficients. For temperature sensors calibrated before 1/1/95 use

a, b, c, d, f0 coefficients, and for temperature sensors calibrated during

and after 1/1/95 use g, h, i, j, f0 coefficients.

- Add choices to conductivity computation: using a, b, c, d, m or using g, h,

i, j, tc, pc coefficients. For temperature sensors calibrated before 1/1/95 use

a, b, c, d, m coefficients, and for temperature sensors calibrated during

and after 1/1/95 use g, h, i, j, tc, pc coefficients.

Most SEASOFT Programs:

---------------------

- Add an new feature: press <Ctrl_F10> to view the input data file's header.

The input data files, which contain headers, are files with extensions such

as .dat, .hex, .ros and .cnv.

All SEASOFT Programs:

--------------------

- Add support for WET Labs AC-3, A-3, and WETStar. Add three new voltage

channels - chlorophyll absorption, beam transmission, and WETStar fluorometer.

Also add new WET Labs variables - chlorophyll concentration, chlorophyll

absorption, beam attenuation, beam transmission and WETStar chlorophyll

concentration.  Normally the values of chlorophyll absorption, chlorophyll

concentration and WETStar chlorophyll concentration are plotted with logarithm

axis, but if -l option is used (in SEASAVE or SEAPLOT), these values are plotted

with linear axis.

NOTES ON SEASOFT VERSION 4.209

==============================

1. BUG FIXES:

FILTER:

------

- It always uses TEMP.CNV as the temporary file even when -t<tempFile> command

line option is used.

SEASAVE:

-------

- The mark variable: mark line type and line color are not saved when updating the

.dsp file during editing the main setup form.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Change variable units for WETLabs chlorophyll concentration and WETStar chlorophyll

concentration from mg/m^3 to �g/l.

- In SEACON, allow the user to enter conductivity tcor and pcor. In DATCNV, add

the conductivity pressure correction - cpcor value next to conductivity sensor

information entry in the output .cnv file's header.

All TERM Programs:

-----------------

- Add 19200 and 38400 to all terminal programs' baud rate choices.

NOTES ON SEASOFT VERSION 4.210

==============================

1. BUG FIXES:

TERMAFM:

-------

- Did not get pressure coefficients from .con file correctly. This problem was first

introduced in TERMAFM V4.208.

2. MODIFICATIONS:

DATCNV, SEACON, SEASAVE:

-----------------------

- In SEACON, eliminate AD590M and AD590B fields from the digiquartz with T/C

coefficients setup form for SBE16 and SBE19. In SEASAVE and DATCNV, set

AD590M and AD590B to default values for digiquartz sensors with T/C for SBE16

and SBE19.

SEACON:

------

- Use separate calibration dates for temperature sensors using a,b,c,d coefficients

and using g,h,i,j coefficients. Also, use separate calibration dates for conductivity

sensors using a,b,c,d coefficients and using g,h,i,j coefficients.

NOTES ON SEASOFT VERSION 4.211

==============================

1. BUG FIXES:

ALIGNCTD:

--------

- An advance time of zero specified on the command line does not override the

value in the Set-Up Form.

TERMAFM:

-------

- When uploading data to .afm files, "Receiving Error..." is reported

when no error is detected.

WFILTER:

-------

- When running Median filter on some .cnv files which were processed by WildEdit

and/or AlignCTD and/or LoopEdit, the program sometimes hung.

2. MODIFICATIONS:

All TERM Programs:

-----------------

- Enable them to upload cast numbers from 0 to 999 instead of 0 to 99.

DATCNV, SEASAVE:

---------------

- Add command line option -r<degrees> to specify the suns angle in degrees from vertical

for surface par or corrected par's computation.

NOTES ON SEASOFT VERSION 4.212

==============================

1. BUG FIXES:

SEACON:

------

- When the pressure type is changed from NONE or Strain Gauge to Digiquartz w/ or w/o TC

or vice versa for SBE16, the number of frequencies in the DataFormat subform was not

updated accordingly.

2. NEW FEATURES:

SEASAVE, DATCNV, DERIVE, ASCIIIN:

--------------------------------

- Add an new variable - Potential Temperature Anomaly.

SEASAVE, SEAPLOT:

----------------

- Add a new command line option -bc<colorIndex> to let the user select the background

color for MultiXY plot.  The color index range is [0, 14] and the color map is the same

SEAPLOT batch mode color selection.  The default background color is black.  If a

variable has the same color as the background color, the variable's color is changed

to black.  This option was added to support screen dumps to color printers via Windows.

SEASAVE (realtime):

------------------

- Add an new choice to the Store Lat/Lon in Raw Data File menu: "Append to Every Scan and

Append to .NAV File when <Ctrl_N> is Pressed".

NOTES ON SEASOFT VERSION 4.213

==============================

1. BUG FIXES:

SEASAVE:

-------

- When displaying archived data, if the screen is dumped after reaching the

end of the data set status line values other than pressure, temperature,

and salinity are incorrect.

- When acquiring data from a SBE 9/11 with a RS-232 interface if the

computer was not keeping up with the incoming data SEASAVE was not sending

XOFF commands correctly to the deck unit causing some characters

to be dropped.  This occurs only if SEASAVE is configured for use with

a water sampler.  One side effect is that non-existing bottle confirmations

were displayed.

- If bottles were fired rapidly (less than 3 seconds between bottles) some

bottle confirmations were not detected by SEASAVE.  When firing sequentially,

if one confirmation was missed subsequent confirmations were not detected.

2. MODIFICATIONS:

SEACON, SEASAVE:

---------------

- Move the "Min Pressure to Enable Altimeter Alarm [db] = " field from

SEASAVE's ASCII Output Set Up form to SEACON's Altimeter Constants Set Up form.

SEASAVE:

-------

- Keep Set-Altimeter-Alarm-On commands to the deck unit at least two

seconds apart.

- Set Column #0 in the Marked Variable List in the Misc Run Parameters

form to Scan Number, and make this field fixed (users cannot modify it).

This insures that scan numbers are always included in .mark files.

NOTES ON SEASOFT VERSION 4.214

==============================

1. BUG FIXES:

ALIGNCTD:

--------

- If the total number of scans in the input was less than the window size,

extra scans with incorrect values were added to the file.

SEASAVE:

-------

- When the units were changed from Metric to English or vice versa, some of

variables descriptions for X-Y overlay display were garbled.

- If Lat/Lon data is added by the NMEA interface, and the display mode is

not set to X-Y overlay display, the Lat/Lon data stored in .NAV file was

not updated when <Ctrl-N> was pressed.

SEASOFT:

-------

- When using the -a command line option to process all files, if one of the

modules aborts with a partially processed TEMP.CNV file, the partially

processed TEMP.CNV file caused errors for subsequent processing.

2. MODIFICATIONS:

SEASOFT:

-------

- Modify the batch processing so that if one of the modules encounters an error

it will display the error message and wait for the user to press a key to continue.

Processing resumes with the next file in the file list.  Prior to this version

processing would resume with the next module in the batch file (the remaining files

in the list were not processed).

3. NEW FEATURES:

DERIVE, SEASAVE:

---------------

- Add DTDT (delta temperature delta time) to DERIVE and add an optional DTDT's

window size in seconds to the existing command line option -v1,<windowSize>.

If the window size is not specified, the default window size of 300 seconds

will be used.

SEACON:

------

- Add support for surface PAR data to be appended by the SBE 33 or SBE 36 deck

unit with NEMA interface (for SBE 19 and SBE 25).

NOTES ON SEASOFT VERSION 4.215

==============================

1. BUG FIXES:

BINAVG:

- If the command line option -c is used to set the first bin's center value and

surface bin is set to yes and the first bin minimum is greater than the surface

bin maximum then the surface bin maximum was incorrectly reset to the binsize / 2.

DATCNV:

------

- For SBE25, if the user chose to obtain Bottle Firing data from .AFM file and

if the SBE25 internal recording rate and AFM real time rate were not the same,

DATCNV didi not extract correct scans from the .HEX file into the .ROS file.

- If the AFM real time data rate is 4 or 8 scans per second, DATCNV incorrectly

displayed a "Scans Don't Match" error, and the scan ranges stored in the .BL file

were incorrect.

TERM1621, TERM17, TERM19, TERM25:

- The command line option -f<upldFileName> used to run the batch process for

uploading data did not work.

2. NEW FEATURES:

SEAPLOT:

-------

- Add a choice of Scalable Dots to Plot Line Type's choices.

SEASAVE:

-------

- Add Altimeter Height + Depth and Altimeter Height + Pressure to Remote Display

Data Type's choices. If either of these choices is chosen, display Altimeter

Height and Depth or Pressure alternately.

NOTES ON SEASOFT VERSION 4.216

==============================

1. BUG FIXES:

All SEASOFT Programs:

--------------------

- When the input and output filenames are the same and the output

file is on a different drive than where the SEASOFT module is executing then

the output file is not created and the TEMP.CNV file is not removed.

2. MODIFICATIONS:

DATCNV:

------

- Modify the program so that the selected variable list is retained

when the input path is changed or a different .con file is selected.

3. NEW FEATURES:

SEACON, SEASAVE:

---------------

- Add "Lat/Lon Data Added by NMEA Interface" option to sbe911's

setup form.

TERM1621:

--------

- Add an option in the communication set up form to select either

full duplex RS232 or half duplex RS485.

NOTES ON SEASOFT VERSION 4.217

==============================

1. BUG FIXES:

DATCNV:

------

- A divide by zero runtime math error occures with instrument type SBE19

when get water bottle data from .afm file is selected.

SEASAVE (realtime):

------------------

- The SBE 11 Deck Unit was programmed to transmit 4 bytes per output word.

Since SEASAVE expects 3 bytes per word the displayed data is garbage.

This bug applies only to SBE 11 deck units with firmware versions less than 2.0

with SEASOFT V 4.202 through 4.216.

This bug does not affect any SBE 11plus deck units.

2. NEW FEATURES:

All SEASOFT Programs:

--------------------

- Add a new voltage output sensor: FGP pressure. Up to eight FGP sensors

can be used on the CTD at once.

- Add a new voltage output sensor: Seapoint Turbidity Meter.

SEASAVE (realtime):

------------------

- Add a command line option -u3 to enable control of storing realtime data

to disk. When -u3 is specified, if bit3 of the middle byte of the modulo

word is 0 store data, otherwise don't.

- Add a new variable "Storing Data" to indicate if the current scan is stored

on to the disk or not.

NOTES ON SEASOFT VERSION 4.218

==============================

1. BUG FIXES:

DATCNV, SEASAVE:

---------------

- Conductivity and salinity values from wide-range conductivity sensors using

ghij coefficients were incorrect.

DATCNV, SEASAVE, Terminal Programs:

----------------------------------

- When the user enters words such as "cast" and "hdr" into the user-entered header

information, the program would get incorrect information from the header part

of the data file. For instance, the start time for the data set would be wrong.

SEAPLOT:

-------

-  Overlay plots on printer are not using the correct line types (connected

line or dots).

-  The whole set of data for the overlay display was plotted even when the screen

display contains partial data set.

2. MODIFICATIONS:

All SEASOFT Programs:

--------------------

- Add choices for primary and secondary sensors to Seapoint Turbidity Meter sensor.

NOTES ON SEASOFT VERSION 4.218a

==============================

1. NEW FEATURES:

SEASAVE:

-------

- Add a command line option -fb to enable bottle firing detection for GO 1015 when the

Firing Bottle Type is set to NONE.  This is useful when both serial ports are used

for other functions and the bottles are fired with the deckunit.

NOTES ON SEASOFT VERSION 4.219

==============================

1. BUG FIXES:

DATCNV:

------

- When .hex files contain extra blanks at the end of scans, DATCNV reports

scan length error for these scans and doesn't convert them.

2. MODIFICATIONS:

DATCNV, SEASAVE:

---------------

- Don't match sensor serial numbers for archived data display for sbe31.

SEASAVE:

-------

- If the GPS NMEA message (such as GGA or GLL message) contains only the time

(hhmmss) but not he month-date information, the time information will still

be saved in the output file's header as the NMEA UTC time.

- Retain the user entered header information in the setup form.

3. NEW FEATURES:

SEASAVE:

-------

- Add a command line option -ss to enable the user to delay saving data to the

output file.  Initially, the realtime data will be displayed on the screen but

not saved.  When the <Ctrl-D> key is pressed data is saved to the output

file.  If X-Y display is selected, the display will be cleared when <Ctrl-D>

is pressed.

- Save the entire command line to output file's header.

NOTES ON SEASOFT VERSION 4.220

==============================

1. BUG FIXES:

CELLTM:

------

- The corrected conductivity values were incorrect when the conductivity unit

were either milli Siemens/cm or micro Siemens/cm.

SEASAVE:

-------

- Altimeter's alarm did not work for SEASOFT versions 4.213 to 4.219 when using

the IEEE-488 interface.

2. MODIFICATIONS:

DATCNV:

------

- Modify DATCNV's batch process so that it uses the same base file name as the .CNV

file when getting bottle firing information from .BL files.

SEACON:

------

- Modify the setup form for temperature and conductivity calibration constants

to make it obvious which set of calibration coefficients is selected.

3. NEW FEATURES:

ALIGNCTD:

--------

- Add a new feature to allow the user to align WETStar Fluorometer values

relative to pressure values.

All SEASOFT Programs:

--------------------

- Add a new type of sensor, Dr. Haardt Yellow Substance, and add a new field -

Gain Range Switch to the Dr. Haardt sensor's setup form.  Users can select

the gain based on the Output Voltage Level, the Modulo Bit, or NONE.

NOTES ON SEASOFT VERSION 4.221

==============================

1. BUG FIXES:

DATCNV, SEACON, SEASAVE:

-----------------------

- When reading old .CON files SEASOFT assumed that the temperature and conductivity

coefficients were G,H,I,J instead of A,B,C,D.  The G,H,I,J coefficients are undefined

in old versions of .CON files.  This bug is only in V 4.220.

NOTES ON SEASOFT VERSION 4.222

==============================

1. BUG FIXES:

BUOYANCY:

--------

- When processing several files in the same session, the output .CNV

file from the first file was correct.  The subsequent .CNV files were

incorrect.

2. MODIFICATIONS:

CELLTM:

------

- Remove command line options -s and -2 and reorganize the setup form.

TERMAFM:

-------

- Bottle positions can be selected in the range of 1 and 36 rather than being limited

to the total number of bottles.