CTD ACQUISITION

Be sure LADCP is set up and ready to go!

SEASAVE

Acquire Data

Change Sta #:   401-1### (### = 3 digit sta #)

Choose .DSP file:

OC401DP.DSP  (Pmax=3500)

OC401MD.DSP (Pmax=1000)

OC401SH.DSP  (Pmax=300)

F10 :  start acquisition

Record position info on log sheet at surface

(pressure, time, lat, lon, depth,  etc)

Lower CTD to just below surface to start pumps.

When deck unit reads 0011 instead of 0010 ...

bring CTD back to surface and

STOP there before starting back down.

Stop winch at bottom

<ctrl> F3 : to fire bottle

(be careful because <ctrl>F1 ends acquisition)

Record bottle firing P,T,S on log sheet

Record  pressure, time, lat, lon, depth, etc. on log sheet

Record altimeter reading and wire out on log sheet

<ctrl> F3 : to fire rest of bottles during uptrace

Record bottle firing P,T,S on log sheet

BACK ON DECK:		<CTRL> F1 :  end acquistion

<ESC>

<ESC>

Quit without saving

Exit = Yes

NEXT ... AFTER CTD ACQUISITION

SEASAVE

Display Archived Data

Choose .DSP file:

SCANCHK.DSP ( for finding start scan of downtrace)

(may have to change scan # range in SCANCHK.DSP)

F10 :  start display

Record how many scans to skip on log sheet

(before CTD starts downtrace after returning to the surface)

<ctrl> F1 : to stop displaying

Choose .DSP file: to change back to original dsp file

OC401??.DSP ( for plotting)

F10 : to plot ctd trace

<ctrl> F9 : to print it out

<ctrl> F1 : to stop

<ESC>

<ESC>

<ESC>

Exit = Yes

NOW RUN BATCH FILES TO DO Q/C EDITS AND ARCHIVE DATA

w_ctd OC401 401-1      ###               ##

(3 digit sta #)       (scans to skip)

w_ladc OC401 401-1   ###

(3 digit sta #)     (do not pass 4th argument)